LABORATORY MANUAL

BASIC SIMULATION

II B.TECH -I Semester (ECE)

# MARRI EDUCATIONAL SOCIETY’S GROUP OF INSTITUTIONS

Dundigal, Quthbullapur (M), Hyderabad-500043.

Dundigal, Quthbullapur (M), Hyderabad-500043.

DEPARTMENT OF ELECTRONICS & COMMUNICATION ENGINEERING

List of Experiments:

1. Basic Operations on Matrices.

2. Generation of Various Signals and Sequences (Periodic and Aperiodic), such as

Unit impulse, unitstep, square, saw tooth, triangular, sinusoidal, ramp, sinc.

3. Observations on signals and sequences such as addition, multiplication, scaling ,

shifting, folding,computation of energy and average power.

4. Finding the even and odd parts of signal/ sequence and real and imaginary parts

of signal.

5. Convolution between signals and sequences.

6. Autocorrelation and cross correlation between signals and sequences.

7. Verification of linearity and time invariance properties of a given continuous/

discrete system.

8. Computation of unit sample, unit step and sinusoidal responses of the given

LTI system andverifying its physical realizability and stability properties.

9. Gibbs phenomenon.

10. Finding the Fourier transform of a given signal and plotting its magnitude and

Phase spectrum.

11. Waveform synthesis using Laplace Transform.

12. Locating the zeros and poles and plotting the pole-zero maps in S plane and

Z-plane for the giventransfer function.

13. Generation of Gaussian noise (real and complex), computation of its mean,

M.S.Value and its Skew,Kurtosis, and PSD, Probability Distribution Function.

14. Sampling theorem verification.

15. Removal of noise by autocorrelation / cross correlation.

16.Extraction of periodic signal masked by noise using correlation.

17.Verification of winer-khinchine relations.

18. Checking a random process for stationary in wide sense.

FG

INSTRUCTIONS TO THE STUDENT FJHINSTRUCTIONS TO THE STUDENT

FJHFHINSTRUCTIONS TO THE STUDENT

- Students are required to attend all labs.

- Students will work individually in hardware laboratories and in computer laboratories.

- While coming to the lab bring the lab manual cum observation book, record etc.

- Take only the lab manual, calculator (if needed) and a pen or pencil to the work area.

- Before coming to the lab, prepare the prelab questions. Read through the lab experiment to familiarize yourself with the components and assembly sequence.

- Utilize 3 hours time properly to perform the experiment (both in software and hardware) and note down the readings properly. Do the calculations, draw the graph and take signature from the instructor.

- If the experiment is not completed in the prescribed time, the pending work has to be done in the leisure hour or extended hours.

- You have to submit the completed record book according to the deadlines set up by your instructor.

- For practical subjects there shall be a continuous evaluation during the semester for 25 sessional marks and 50 end examination marks.

- Of the 25 marks for internal, 15 marks shall be awarded for day-to-day work and 10 marks to be awarded by conducting an internal laboratory test.

INDEX

| SL.NO. | EXPERIMENT NAME | PAGENO. |
| --- | --- | --- |
| 1 | Basic Operations on Matrices. | 5 |
| 2 | Generation of Various Signals and Sequences (Periodic and Aperiodic), such as Unit impulse, unit step, square, saw tooth, triangular, sinusoidal, ramp, sinc. | 10 |
| 3 | Observations on signals and sequences such as addition, multiplication, scaling ,shifting, folding, computation of energy and average power. | 28 |
| 4 | Finding the even and odd parts of signal/ sequence and real and imaginary parts of signal. | 39 |
| 5 | Convolution between signals and sequences. | 45 |
| 6 | Autocorrelation and cross correlation between signals and sequences. | 50 |
| 7 | Verification of linearity and time invariance properties of a given continuous/discrete system. | 59 |
| 8 | Computation of unit samples, unit step and sinusoidal response of the given LTI system and verifying its physical realiazability and stability properties. | 66 |
| 9 | Gibbs phenomenon. | 72 |
| 10 | Finding the Fourier transform of a given signal and plotting its magnitude and Phase spectrum. | 76 |
| 11 | Waveform synthesis using Laplace Transform. | 81 |
| 12 | Locating the zeros and poles and plotting the pole-zero maps in S plane and Z-plane for the given transfer function. | 84 |
| 13 | Generation of Gaussian noise (real and complex), computation of its mean, M.S. Value and its Skew, Kurtosis, and PSD, Probability Distribution Function. | 90 |
| 14 | Sampling theorem verification. | 94 |
| 15 | Removal of noise by autocorrelation / cross correlation. | 104 |
| 16 | Removal of noise by auto correlation/cross correlation. | 111 |
| 17 | Verification of winer- khinchine relations. | 116 |
| 18 | Checking a random process for stationary in wide sense. | 120 |

EXPERMENT NO:1

BASIC OPERATIONS ON MATRICES

AIM: -

To write a MATLAB program to perform some basic operation on matrices

such as addition, subtraction, multiplication.

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

MATLAB, which stands for MATrix LABoratory, is a state-of-the-art mathematical

software package, which is used extensively in both academia and industry. It is an

interactive program for numerical computation and data visualization, which along with its

programming capabilities provides a very useful tool for almost all areas of science and

engineering. Unlike other mathematical packages, such as MAPLE or MATHEMATICA,

MATLAB cannot perform symbolic manipulations without the use of additional Toolboxes.

It remains however, one of the leading software packages for numerical computation.

As you might guess from its name, MATLAB deals mainly with matrices. A scalar is

a 1-by-1 matrix and a row vector of length say 5, is a 1-by-5 matrix.. One of the many

advantages of MATLAB is the natural notation used. It looks a lot like the notation that you

encounter in a linear algebra. This makes the use of the program especially easy and it is what

makes MATLAB a natural choice for numerical computations. The purpose of this

experiment is to familiarize MATLAB, by introducing the basic features and commands of

the program.

Built in Functions:

1. Scalar Functions:

Certain MATLAB functions are essentially used on scalars, but operate element-wise when

applied to a matrix (or vector). They are summarized below.

1. sin - trigonometric sine

2. cos - trigonometric cosine

3. tan - trigonometric tangent

4. asin - trigonometric inverse sine (arcsine)

5. acos - trigonometric inverse cosine (arccosine)

6. atan - trigonometric inverse tangent (arctangent)

7. exp - exponential

8. log - natural logarithm

9. abs - absolute value

10. sqrt - square root

11. rem - remainder

12. round - round towards nearest integer

13. floor - round towards negative infinity

14. ceil - round towards positive infinity

2. Vector Functions:

Other MATLAB functions operate essentially on vectors returning a scalar value. Some of

these functions are given below.

1. max largest component : get the row in which the maximum element lies

2. min smallest component

3. length length of a vector

4. sort sort in ascending order

5. sum sum of elements

6. prod product of elements

7. median median value

8. mean mean value std standard deviation

3. Matrix Functions:

Much of MATLAB‟ s power comes from its matrix functions. These can be further separated

into two sub-categories.

The first one consists of convenient matrix building functions, some of which are given

below.

1. eye - identity matrix

2. zeros - matrix of zeros

3. ones - matrix of ones

4. diag - extract diagonal of a matrix or create diagonal matrices

5. triu - upper triangular part of a matrix

6. tril - lower triangular part of a matrix

7. rand - randomly generated matrix

eg: diag([0.9092;0.5163;0.2661])

ans =

0.9092 0 0

0 0.5163 0

0 0 0.2661

commands in the second sub-category of matrix functions are

1. size size of a matrix

2. det determinant of a square matrix

3. inv inverse of a matrix

4. rank rank of a matrix

5. rref reduced row echelon form

6. eig eigenvalues and eigenvectors

7. poly characteristic polynomial

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

clc;

close all;

clear all;

a=[1 2 -9 ; 2 -1 2; 3 -4 3];

b=[1 2 3; 4 5 6; 7 8 9];

disp('The matrix a= ');

a

disp('The matrix b= ');

b

% to find sum of a and b

c=a+b;

disp('The sum of a and b is ');

c

% to find difference of a and b

d=a-b;

disp('The difference of a and b is ');

d

%to find multiplication of a and b

e=a*b;

disp('The product of a and b is ');

e

\

OUTPUT:-

The matrix a=

a =

1     2    -9

2    -1     2

3    -4     3

The matrix b=

b =

1     2     3

4     5     6

7     8     9

The sum of a and b is

c =

2     4    -6

6     4     8

10     4    12

The difference of a and b is

d =

0     0   -12

-2    -6    -4

-4   -12    -6

The product of a and b is

e =

-54   -60   -66

12    15    18

8    10    12

RESULT:-

Finding addition, subtraction, multiplication using MATLAB was

Successfully completed.

VIVA QUESTIONS:-

1. Define Signal?

2. Define determistic and Random Signal?

3. Define Delta Function?

4. What is Signal Modeling?

5. Define Periodic and a periodic Signal?

6.Write the relation between Step and Impulse signals?

7. Write the relation between Ramp and impulse signals?

8. Write the relation between Ramp and Step signals?

9.Define Standerd signals?

10.Give examples of Standerd signals?

11. Define Continuous and Discrete Time Signals?

12. What are the Different types of representation of discrete time signals?

13. What are the Different types of Operation performed on signals?

14. Define Symetric and Anti-Symmetric Signals?

15. What is the importance of Linear Systems?

16.Relation between impulse,step and ramp?

17.Examplels for power signals?

18. Examplels for energy signals?

19.Characteristics of dynamic system?

20 Characteristics of static system?

21. What is linear system?

22.What is causal system?

23.What is causal signal?

24.What is a stable system?

25.What is a LTI system?

26. Relation between ramp and parabolic signal?

27.Define orthogonality between signals?

28. State properties of convolution?

29.Give some examples of causal system?

29. State superposition theorem?

30.Define transfer fuction?

31.State commutative property of convolution?

32. State associative property of convolution?

33. How the DT system is represented?

34.What is folding operation?

35.What is shifting operation?

36.What is time scaling operation?

37. What is SINC function?

38. What is amplitude scaling operation?

39.What is sampling function?

40. What is summation operation?

41. What is multiplication operation?

42. Give some examples of linear system?

43 Give some examples of stable system?

44. Give some examples of dynamic system?

45. Give some examples of static system?

46. Give some examples of random signal?

47. Give some examples of feedback system?

48.Give conditions for distortionless transmission system?

49.Write expression for rectangular function?

50. Write expression for tringular function?

EXERCISE PROGRAMS

1. Enter the matrix M=[1,3,-1,6:2,4,0,-1;0,-2,3,-1;-1,2,-5,1] and N=[-1,-3,3;2,-1,6;1,4,-1;2,-1,2], perform multiplication on M and N. Can order of multiplication be switched? Why or why not?

2. Enter the matrix

M = [1,-2,8,0]  and N =  [1   5  6  8; 2 5 6 9]

Perform addition on M and N and see how matlab reacts.

3. Find the transpose of null matrix using matlab

4. Write a MATLAB program to perform the division operation on the following matrix

A = [24,-30, 64,-81], b= [6,5,8,9]  and verify the result.

5. Write a matlab program to perform addition operation using 2x3 matrix. Assume any numbers

6. Enter the matrix

A = [1 6 9 8 5; 9 3 5 8 4; 5 6 3 5 7],  B = [6 5 9 3 5; 6 5 4 8 5; 6 3 5 7 9],

C = [2 5 9 3 4; 5 6 3 7  8; 9 8 6 5 4]

Find [(A+B)+C]T

7. Enter the matrix

A = [1 6 9 8 5; 9 3 5 8 4; 5 6 3 5 7],  B = [6 5 9 3 5; 6 5 4 8 5; 6 3 5 7 9],

C = [2 5 9 3 4; 5 6 3 7  8; 9 8 6 5 4]

Find [(A-B)+C]-1

8. Write a matlab program to perform addition operation using 3x2 matrix. Assume any       numbers

9. Write a MATLAB program to perform the division operation on the following matrix

A = [25,-35, 121,-21], b= [5,5,11,3]  and perform the transpose function on the answer

10. Find the addition of null matrix and unity matrix of order 3x3.

11. Write a MATLAB program to find rank of the matrix A= [6 5 9 3 5; 6 5 4 8 5; 6 3 5 7 9].

12. Write a MATLAB program to find Eigen of the matrix A= [2 5 9 3 4; 5 6 3 7  8; 9 8 6 5 4].

13. Write a MATLAB program to find inverse of the matrix A= [1 6 9 8 5; 9 3 5 8 4; 5 6 3 5 7]

14. Write a MATLAB program to find determinant of the matrix

A= [1 6 9 8 5; 9 3 5 8 4; 5 6 3 5 7].

15. Write a MATLAB program to find Transpose of the matrix

A=[1 6 9 8 5; 9 3 5 8 4; 5 6 3 5 7]

16.  Find the multiplication of null matrix and unity matrix of order 3x3.

17. Write a matlab program to perform multiplication operation using 2x3 matrix. Assume any numbers.

18. Write a MATLAB program to perform the division operation on the following matrix

A = [2,-5, 21,-21], b= [5,15,11,13].

19. Enter the matrix

A = [6 9 8 5;3 5 8 4;6 3 5 7],  B = [5 9 3 5;5 4 8 5;3 5 7 9],

C = [5 9 3 4;6 3 7  8;8 6 5 4]

Find [(A-B)+C]-1

20. Write a matlab program to perform division operation using 3x2 matrix. Assume any       numbers

21. Write a MATLAB program to perform the division operation on the following matrix

A = [5,-3,11,-1], b= [6,5,1,13]  and perform the transpose function on the answer.

22. Find the addition of null matrix and Identity matrix of order 4x4.

23. Enter the matrix M=[2,13,-11;12,4,2;-2,3,-1] and N=[2,-1,6;1,4,-1;2,-1,2], perform multiplication on M and N.

24. Enter the matrix M = [-2,8,0]  and N =  [1   5  6  8; 2 5 6 9]

Perform addition and multiplication on M and N and see how matlab reacts.

25. Enter the matrix A = [6 9 8 5;3 5 8 4;6 3 5 7],  B = [5 9 3 5;5 4 8 5;3 5 7 9],

C = [2 5 9 3 4; 5 6 3 7  8; 9 8 6 5 4]

Find [(A+B)+C]T

Real Time application:

We see the results of matrix mathematics in every computer-generated image that has a reflection, or distortion effects such as light passing through rippling water.

EXPERMENT NO:2

GENERATION OF VARIOUS SIGNALS & SEQUENCES

AIM:-   To write a “MATLAB” Program to generate various signals and sequences,such as

unit impulse, unit step, unit ramp, sinusoidal,square,saw tooth,triangular,sinc signals.

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

One of the more useful functions in the study of linear systems is the "unit impulse function."   An ideal impulse function is a function that is zero everywhere but at the origin, where it is infinitely high.  However, the area of the impulse is finite.  This is, at first hard to visualize but we can do so by using the graphs shown below.

##### Key Concept: Sifting Property of the Impulse

##### If b>a, then

##### Example: Another integral problem

Assume a<b, and evaluate the integral

Solution:

We now that the impulse is zero except at t=0 so

And

Unit Step Function

The unit step function and the impulse function are considered to be fundamental functions in engineering, and it is strongly recommended that the reader becomes very familiar with both of these functions.

The unit step function, also known as the Heaviside function, is defined as such:

Sometimes, u(0) is given other values, usually either 0 or 1. For many applications, it is irrelevant what the value at zero is. u(0) is generally written as undefined.

### Derivative

The unit step function is level in all places except for a discontinuity at t = 0. For this reason, the derivative of the unit step function is 0 at all points t, except where t = 0. Where t = 0, the derivative of the unit step function is infinite.

The derivative of a unit step function is called an impulse function. The impulse function will be described in more detail next.

### Integral

The integral of a unit step function is computed as such:

## Sinc Function

There is a particular form that appears so frequently in communications engineering, that we give it its own name. This function is called the "Sinc function" and is discussed below:

The Sinc function is defined in the following manner:

And

Sinc(0)=1

The value of sinc(x) is defined as 1 at x = 0, since

This fact can be proven by noting that for x near 0,

Then, since cos(0) = 1, we can apply the Squeeze Theorem to show that the sinc function approaches one as x goes to zero. Thus, defining sinc(0) to be 1 makes the sinc function continuous.

Also, the Sinc function approaches zero as x goes towards infinity, with the envelope of sinc(x) tapering off as 1/x.

## Rect Function

The Rect Function is a function which produces a rectangular-shaped pulse with a width of 1 centered at t = 0. The Rect function pulse also has a height of 1. The Sinc function and the rectangular function form a Fourier transform pair.

A Rect function can be written in the form:

where the pulse is centered at X and has width Y. We can define the impulse function above in terms of the rectangle function by centering the pulse at zero (X = 0), setting it's height to 1/A and setting the pulse width to A, which approaches zero:

We can also construct a Rect function out of a pair of unit step functions

Here, both unit step functions are set a distance of Y/2 away from the center point of (t - X).

SAW TOOTH:-

The sawtooth wave (or saw wave) is a kind of non-sinusoidal waveform. It is named a sawtooth based on its resemblance to the teeth on the blade of a saw. The convention is that a sawtooth wave ramps upward and then sharply drops. However, there are also sawtooth waves in which the wave ramps downward and then sharply rises. The latter type of sawtooth wave is called a 'reverse sawtooth wave' or 'inverse sawtooth wave'. As audio signals, the two orientations of sawtooth wave sound identical. The piecewise linear function based on the floor function of time t, is an example of a sawtooth wave with period 1.

Triangle wave

A triangle wave is a non-sinusoidal waveform named for its triangular shape.A bandlimited triangle wave pictured in the time domain (top) and frequency domain (bottom). The fundamental is at 220 Hz (A2).Like a square wave, the triangle wave contains only odd harmonics. However, the higher harmonics roll off much faster than in a square wave (proportional to the inverse square of the harmonic number as opposed to just the inverse).It is possible to approximate a triangle wave with additive synthesis by adding odd harmonics of the fundamental, multiplying every (4n−1)th harmonic by −1 (or changing its phase by π), and rolling off the harmonics by the inverse square of their relative frequency to the fundamental.This infinite Fourier series converges to the triangle wave:

Sinusoidal Signal Generation

The sine wave or sinusoid is a mathematical function that describes a smooth repetitive oscillation. It occurs often in pure mathematics, as well as physics, signal processing, electrical engineering and many other fields. Its most basic form as a function of time (t) is:

where:

• A, the amplitude, is the peak deviation of the function from its center position.

• ω, the angular frequency, specifies how many oscillations occur in a unit time

interval, in radians per second

• φ, the phase, specifies where in its cycle the oscillation begins at t = 0.

A sampled sinusoid may be written as:

where f is the signal frequency, fs is the sampling frequency, θ is the phase and A is the

amplitude of the signal.

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

%unit impulse function%

clc;

clear all;

close all;

t=-10:1:10;

x=(t==0);

subplot(2,1,1);

plot(t,x,'g');

xlabel('time');

ylabel('amplitude');

title('unit impulse function');

subplot(2,1,2);

stem(t,x,'r');

xlabel('time');

ylabel('amplitude');

title('unit impulse discreat function');

%unit step function%

clc;

clear all;

close all;

N=100;

t=1:100;

x=ones(1,N);

subplot(2,1,1);

plot(t,x,'g');

xlabel('time');

ylabel('amplitude');

title('unit step function');

subplot(2,1,2);

stem(t,x,'r');

xlabel('time');

ylabel('amplitude');

title('unit step discreat function');

%unit ramp function%

clc;

clear all;

close all;

t=0:20;

x=t;

subplot(2,1,1);

plot(t,x,'g');

xlabel('time');

ylabel('amplitude');

title('unit ramp function');

subplot(2,1,2);

stem(t,x,'r');

xlabel('time');

ylabel('amplitude');

title('unit ramp discreat function');

%sinusoidal function%

clc;

clear all;

close all;

t=0:0.01:2;

x=sin(2*pi*t);

subplot(2,1,1);

plot(t,x,'g');

xlabel('time');

ylabel('amplitude');

title('sinusoidal signal');

subplot(2,1,2);

stem(t,x,'r');

xlabel('time');

ylabel('amplitude');

title('sinusoidal sequence');

%square function%

clc;

clear all;

close all;

t=0:0.01:2;

x=square(2*pi*t);

subplot(2,1,1);

plot(t,x,'g');

xlabel('time');

ylabel('amplitude');

title('square signal');

subplot(2,1,2);

stem(t,x,'r');

xlabel('time');

ylabel('amplitude');

title('square sequence');

%sawtooth function%

clc;

clear all;

close all;

t=0:0.01:2;

x=sawtooth(2*pi*5*t);

subplot(2,1,1);

plot(t,x,'g');

xlabel('time');

ylabel('amplitude');

title('sawtooth signal');

subplot(2,1,2);

stem(t,x,'r');

xlabel('time');

ylabel('amplitude');

title('sawtooth sequence');

%trianguler function%

clc;

clear all;

close all;

t=0:0.01:2;

x=sawtooth(2*pi*5*t,0.5);

subplot(2,1,1);

plot(t,x,'g');

xlabel('time');

ylabel('amplitude');

title('trianguler signal');

subplot(2,1,2);

stem(t,x,'r');

xlabel('time');

ylabel('amplitude');

title('trianguler sequence');

%sinc function%

clc;

clear all;

close all;

t=linspace(-5,5);

x=sinc(t);

subplot(2,1,1);

plot(t,x,'g');

xlabel('time');

ylabel('amplitude');

title('sinc signal');

subplot(2,1,2);

stem(t,x,'r');

xlabel('time');

ylabel('amplitude');

title('sinc sequence');

OUTPUT:-

unit impulse function

unit step function

unit ramp function

sinusoidal function

square function

sawtooth function

trianguler function

sinc function

RESULT:-

Thus the Generation of continuous time signals like unit step, sawtooth,

triangular, sinusoidal, ramp and sinc functions are successfully completed

by using  MATLAB.

VIVA QUESTIONS:-

1. Define Symetric and Anti-Symmetric Signals?

2. Define Continuous and Discrete Time Signals?

3. What are the Different types of representation of discrete time signals?

4. What are the Different types of Operation performed on signals?

5. What is System?

6.Explain Even Signals?

7.Explain Odd signals?

8.Define Periodic signals?

9.Define Aperiodic signals?

10.Differentiate Energy and Power signals with examples?

11.What is Energy Signals?

12.What is Power signals?

13.Define Systems?

14..How to determine time period of Periodic signals?

15.Define Frequency of signals?

16.Write matlab command for subtraction of two signals?

17. Write matlab command for addition of two signals?

18. Write matlab command for multiplication of two signals?

19. Write matlab command amplitude scaling of a sequence?

20. Write matlab command time scaling of a sequence?

21. Write matlab command time shifting of a sequence?

22. Write matlab command right shifting of a sequence?

23. Write matlab command left shifting of a sequence?

24.How to calculate even part of a signal?

25. How to calculate odd part of a signal?

26.Write conditionfor physical realization system?

27.What is paley wiener criterion?

28. Write matlab command for division of two signals?

29. Write matlab command for left shift the signal to 8 times of the original signal

30 Write matlab command for right shift the signal to 8 times of the original signal

31. Write matlab command for folding of a sequence?

32. Write matlab command for shift a positive time line signal to negative timeline signal

33. Write matlab command for shift a negative time line signal to positive timeline signal

34. Write matlab command for right shifting of a sequence to 8 times of the original signal

35. Write matlab command for left shifting of a sequence to 8 times of the original signal

36.Define complex exponential signal?

37.Define signum function?

38.Sketch the impulse signal?

39.Sketch the Double sided exponential signal?

40.Define real exponential signal?

41.Define step function?

42.What is the sinusoidal signal?

43.Bring out the analogy between vectors and signals?

44.What is orthonormal vector and orthonormal  set of vectors?

45.Define orthogonal signal space?

46.Define mean square erroe?

47.Explain orthogonal function?

48.What is basis vectors ?

49.The relation ship between unit step function and signum function?

50.Define rectangular function?

EXERCISE QUESTIONS

- Generate rectangular pulse function by using MATLAB.

Write a matlab program to get the above output where t0 = 2

- Write a program to get the result in signal

r(t) = u(t) – 2*u(t+1)

- Write a program to get the following output t = -1: -5

Write  mat lab program to get the above output

X(t) = 1; (0<t<2)

= 10;(2<t<15)   Note: Add ramp and unit

- Write a matlab program to generate a sine wave with amplitude = 3, frequency 20Hz.

- Write a matlab program to generate a cos wave with amplitude = 3, frequency 20Hz.

- Write a matlab program to generate a triangular  wave with amplitude = 8, frequency 10Hz.

- Write a matlab program to generate a square  wave with amplitude = 2, frequency 10kHz.

- Write a matlab program to generate the signum function.

- Write a matlab program to generate exponential growing signal.

- Write a matlab program to generate exponential decaying signal.

- Write a matlab program to generate a triangular wave with amplitude = 6,frequency 1Hz.

- Write a matlab program to generate a square  wave with amplitude = 5, frequency 5kHz.

- Write a matlab program to generate a sawtooth wave with amplitude = 8, frequency5Khz.

- Write a matlab program to generate a sine wave with amplitude = 5, frequency 5Hz.

- Write a matlab program to generate a cos wave with amplitude = 4, frequency 10Hz.

- Write a matlab program to generate a triangular wave with amplitude = 5, frequency 4Hz.

- Write a matlab program to generate a square  wave with amplitude = 1, frequency 20kHz.

- Write a matlab program to generate the signum function.

- Wr ite a matlab program to generate a square wave with amplitude = 4, frequency 5kHz.

- Write a matlab program to generate a triangular  wave with amplitude = 6, frequency 6Hz.

- Write a matlab program to generate a triangular wave with amplitude = 8,frequency 2Hz.

- Write a matlab program to generate a square  wave with amplitude = 7, frequency 5kHz.

- Write a matlab program to generate a sawtooth wave with amplitude =10, frequency 10Khz.

Real Time Applications:

- Industrial control and automation (Control the velocity or position of an object)

- Examples: Controlling the position of a valve or shaft of a motor

EXPERMENT NO:3

OPERATIONS ON SIGNALS & SEQUENCES

AIM:-

To performs operations on signals and sequences such as addition, multiplication,

scaling, shifting, folding, computation of energy and average power.

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

Basic Operation on Signals:

Time shifting: y(t)=x(t-T)The effect that a time shift has on the appearance of a signal

If T is a positive number, the time shifted signal, x (t -T ) gets shifted to the right,

otherwise it gets shifted left.

Signal Shifting and Delay:

Shifting : y(n)={x(n-k)} ; m=n-k; y=x;

Time reversal: Y(t)=y(-t) Time reversal _ips the signal about t = 0 as seen in

Figure 1.

Signal Addition and Substraction :

Addition: any two signals can be added to form a third signal,

z (t) = x (t) + y (t)

Signal Amplification/Attuation :

Multiplication/Divition :

of two signals, their product is also a signal.

z (t) = x (t) y (t)

folding:

y(n)={x(-n)} ; y=fliplr(x); n=-fliplr(n);

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

%Addition and multiplication of two signals%

clc;

clear all;

close all;

t=0:0.001:2;

s1=6*sin(2*pi*5*t);

subplot(4,1,1);

plot(t,s1,'g');

xlabel('time');

ylabel('amplitude');

title('first signal');

s2=8*sin(2*pi*5*t);

subplot(4,1,2);

plot(t,s2,'r');

xlabel('time');

ylabel('amplitude');

title('second signal');

s3=s1+s2;

subplot(4,1,3);

plot(t,s3,'g');

xlabel('time');

ylabel('amplitude');

title('sum of two signals');

s4=s1.*s2;

subplot(4,1,4);

plot(t,s4,'g');

xlabel('time');

ylabel('amplitude');

title('multiplication of two signals');

%Amplitude scaling for signals%

clc;

clear all;

close all;

t=0:0.001:2;

s1=6*sin(2*pi*5*t);

subplot(3,1,1);

plot(t,s1,'g');

xlabel('time');

ylabel('amplitude');

title('sinusoidal signal');

s2=3*s1;

subplot(3,1,2);

plot(t,s2,'r');

xlabel('time');

ylabel('amplitude');

title('amplified signal');

s3=s1/3;

subplot(3,1,3);

plot(t,s3,'g');

xlabel('time');

ylabel('amplitude');

title('attenuated signal');

%Time scaling for signals%

clc;

clear all;

close all;

t=0:0.001:2;

s1=6*sin(2*pi*5*t);

subplot(3,1,1);

plot(t,s1,'g');

xlabel('time');

ylabel('amplitude');

title('sinusoidal signal');

t1=3*t;

subplot(3,1,2);

plot(t1,s1,'r');

xlabel('time');

ylabel('amplitude');

title('compressed signal');

t2=t/3;

subplot(3,1,3);

plot(t2,s1,'g');

xlabel('time');

ylabel('amplitude');

title('enlarged signal');

%Time shifting of a signal%

clc;

clear all;

close all;

t=0:0.001:3;

s1=6*sin(2*pi*5*t);

subplot(3,1,1);

plot(t,s1,'g');

xlabel('time');

ylabel('amplitude');

title('sinusoidal signal');

t1=t+10;

subplot(3,1,2);

plot(t1,s1,'r');

xlabel('time');

ylabel('amplitude');

title('right shift of the signal');

t2=t-10;

subplot(3,1,3);

plot(t2,s1,'g');

xlabel('time');

ylabel('amplitude');

title('left shift of the signal');

%Time folding of a signal%

clc;

clear all;

close all;

t=0:0.001:2;

s=sin(2*pi*5*t);

m=length(s);

n=[-m:m];

y=[0,zeros(1,m),s];

subplot(2,1,1);

plot(n,y,'g');

xlabel('time');

ylabel('amplitude');

title('original signal');

y1=[fliplr(s),0,zeros(1,m)];

subplot(2,1,2);

plot(n,y1,'r');

xlabel('time');

ylabel('amplitude');

title('folded signal');

OUTPUT:-

Addition and multiplication of two signals

Amplitude scaling for signals

Time scaling for signals

Time shifting of a signal

\

Time folding of a signal

RESULT:-

In this experiment the various oprations on signals have been

Performed Using MATLAB have been demonstrated.

VIVA QUESTIONS:-

1. Define Symetric and Anti-Symmetric Signals?

2. Define Continuous and Discrete Time Signals?

3. What are the Different types of representation of discrete time signals?

4. What are the Different types of Operation performed on signals?

5. What is System?

6.Explain Even Signals?

7.Explain Odd signals?

8.Define Periodic signals?

9.Define Aperiodic signals?

10.Differentiate Energy and Power signals with examples?

11.What is Energy Signals?

12.What is Power signals?

13.Define Systems?

14..How to determine time period of Periodic signals?

15.Define Frequency of signals?

16.Write matlab command for subtraction of two signals?

17. Write matlab command for addition of two signals?

18. Write matlab command for multiplication of two signals?

19. Write matlab command amplitude scaling of a sequence?

20. Write matlab command time scaling of a sequence?

21. Write matlab command time shifting of a sequence?

22. Write matlab command right shifting of a sequence?

23. Write matlab command left shifting of a sequence?

24.How to calculate even part of a signal?

25. How to calculate odd part of a signal?

26.Write conditionfor physical realization system?

27.What is paley wiener criterion?

28. Write matlab command for division of two signals?

29. Write matlab command for left shift the signal to 8 times of the original signal

30 Write matlab command for right shift the signal to 8 times of the original signal

31. Write matlab command for folding of a sequence?

32. Write matlab command for shift a positive time line signal to negative timeline signal

33. Write matlab command for shift a negative time line signal to positive timeline signal

34. Write matlab command for right shifting of a sequence to 8 times of the original signal

35. Write matlab command for left shifting of a sequence to 8 times of the original signal

36.Define complex exponential signal?

37.Define signum function?

38.Sketch the impulse signal?

39.Sketch the Double sided exponential signal?

40.Define real exponential signal?

41.Define step function?

42.What is the sinusoidal signal?

43.Bring out the analogy between vectors and signals?

44.What is orthonormal vector and orthonormal  set of vectors?

45.Define orthogonal signal space?

46.Define mean square erroe?

47.Explain orthogonal function?

48.What is basis vectors ?

49.The relation ship between unit step function and signum function?

50.Define rectangular function?

EXERCISE PROGRAMS

1. Write a MATLAB program to generate amplitude scaling of a sequence.

2. Write a MATLAB program to subtract two sinusoidal signals.

3. Write a MATLAB program to subtract and multiply twosinusoidal signals.

4. Write a MATLAB program to right shift the signal to 5 times of the original signal.

5. Write a MATLAB program to left shift the signal to 8 times of the original signal.

6. Write a MATLAB program to add two different signals with 2 <t<5

7. Write a MATLAB program to shift a positive time line signal to negative timeline signal.

8. Write a MATLAB program to subtract co-sinusoidal signals.

9. Write a MATLAB program to subtract two sinusoidal signals

10. Write a MATLAB program to division and multiply two co-sinusoidal signals.

11. Write a MATLAB program to generate time scaling of a sequence.

12. Write a MATLAB program to generate time shifting of a sequence.

13. Write a MATLAB program to generate time folding of a sequence.

14. Write a MATLAB program to generate amplitude scaling of a sequence with amplitude 5.

15. Write a MATLAB program to generate time scaling of a sequence with time 2sec.

16. Write a MATLAB program to add two different signals with 4 <t<8

17. Write a MATLAB program to shift a negative time line signal to positive timeline signal.

18. Write a MATLAB program to subtract sinusoidal signals.

19. Write a MATLAB program to subtract and divide two sinusoidal signals

20. Write a MATLAB program to add and multiply two co-sinusoidal signals.

21. Write a MATLAB program to generate time scaling of a signal.

22. Write a MATLAB program to generate time shifting of a signal.

23. Write a MATLAB program to generate time folding of a signal.

24. Write a MATLAB program to generate amplitude scaling of a sequence with amplitude 10.

25. Write a MATLAB program to generate time scaling of a sequence with time 5sec.

Real Time Applications:

- Stream processing

- Block processing

- Vector processing

EXPERMENT NO: 4

FINDING EVEN AND ODD & REAL AND IMAGINARY PARTS OF SEQUENCES

AIM: -

Program for finding even and odd parts of sequences Using MATLAB Software&

Program for finding real and imaginary parts of sequences Using MATLAB Software

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

Even and Odd Signal

One of characteristics of signal is symmetry that may be useful for signal analysis. Even

signals are symmetric around vertical axis, and Odd signals are symmetric about origin.

Even Signal: A signal is referred to as an even if it is identical to its time-reversed

counterparts; x(t) = x(-t).

Odd Signal: A signal is odd if x(t) = -x(-t).

An odd signal must be 0 at t=0, in other words, odd signal passes the origin.

Using the definition of even and odd signal, any signal may be decomposed into a sum of

its even part, xe(t), and its odd part, xo(t), as follows:

x(t)=xe(t)+xo(t);

x(t)=1/2{x(t)+x(-t)} +1/2{x(t)-x(-t)}

where

xe(t)=1/2{x(t)+x(-t)} &xo(t)=1/2{x(t)-x(-t)}

It is an important fact because it is relative concept of Fourier series. In Fourier series, a

periodic signal can be broken into a sum of sine and cosine signals. Notice that sine

function is odd signal and cosine function is even signal.

ENERGY AND POWER SIGNAL:

A signal can be categorized into energy signal or power

signal: An energy signal has a finite energy, 0 < E < ∞. In other words, energy signals

have values only in the limited time duration. For example, a signal having only one

square pulse is energy signal. A signal that decays exponentially has finite energy, so, it

is also an energy signal. The power of an energy signal is 0, because of dividing finite

energy by infinite time (or length).

If x(t) is a real-valued signal with Fourier transform X(f), and u(f) is the Heaviside step function, then the function:

contains only the non-negative frequency components of X(f). And the operation is reversible, due to the Hermitian property of X(f):

X(f)* denotes the complex conjugate of X(f) .

The inverse Fourier transform of Xa(f) is the analytic signal:

where x^(t) is the Hilbert transform of x(t) and J is the imaginary unit.

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

%Even,odd,real,imaginary parts of a sequences%

clc;

clear all;

close all;

h=input('enter no.of samples');

m=(h-1)/2;

n=-m:m;

x=input('enter sample values');

subplot(4,1,1);

stem(n,x,'g');

xlabel('time');

ylabel('amplitude');

title('original sequence');

xmir=fliplr(x);

subplot(4,1,2);

stem(n,xmir,'r');

xlabel('time');

ylabel('amplitude');

title('folded sequence');

%even part of sequence%

xeven=(x+xmir)/2;

subplot(4,1,3);

stem(n,xeven,'r');

xlabel('time');

ylabel('amplitude');

title('even part of sequence');

%odd part of sequence%

xodd=(x-xmir)/2;

subplot(4,1,4);

stem(n,xodd,'g');

xlabel('time');

ylabel('amplitude');

title('odd part of sequence');

%Real&Imaginary parts of a sequences%

clc;

clear all;

close all;

y=input('enter complex numbers');

yreal=real(y);

disp('real values of y');

yreal

yimag=imag(y);

disp('imaginary values of y');

yimag

OUTPUT:-

Even,odd,real,imaginary parts of a sequences

enter no.of samples5

enter sample values[1 2 3 4 5]

Real&Imaginary parts of a sequences

enter complex numbers[1+6i 2-5i 3 4+3i 5i]

real values of y

yreal =

1     2     3     4     0

imaginary values of y

yimag =

6    -5     0     3     5

RESULT:-

In this experiment even and odd parts of various signals and energy

and power of signals have been calculated Using MATLAB.

VIVA QUESTIONS:-

1. What is the formula to find odd part of signal?

2. What is Even Signal?

3. What is Odd Signal?

4. What is the formula to find even part of signal?

5.What is the difference b/w stem&plot?

6.What is Energy Signals?

7.What is Power signals?

8.Define Systems?

9.How to determine time period of Periodic signals?

10.Define Frequency of signals?

11. Define Continuous and Discrete Time Signals?

12. What are the Different types of representation of discrete time signals?

13. What are the Different types of Operation performed on signals?

14. What is System?

15..Explain Even Signals?

16.Define and explain time variant and time invariant systems?

17.Write the property of even signal?

18.Write the property of odd signal?

19.Write the discrete time signal condition for even signal?

20. Write the discrete time signal condition for odd signal?

21.Write the equation for even symmetry?

22.Write the equation for odd symmetry?

23.Write relation between exponential and trigonometric fourier series?

24.Fourier transform of impulse function?

25.Define linearity and time invariant properties of a system

26. Explain ideal filters

27.What is the step response of a LPF?

28.Explain signal bandwidth

29.Explain system bandwidth

30.What is paley-wiener criterion?

31.What is fourier series?

32.What is fourier transform?

33.What is signum function?

34.What are singularity functions

35.What is gate function

36.What is Gaussian pulse

36.What is quarter wave symmetry

37.What is trigonometric fourier series

38. What is exponential fourier series

39. What is polar form fourier series

40.Define stable and unstable syatem?

41.Write matlab command in order find magnitude

42. Write matlab command in order to find angle

43. Write matlab command in order to find real part

44. Write matlab command in order to find imaginary part

45. Write matlab command in order to get triangular pulse

46. Write matlab command in order to get rectagular pulse

47. Write matlab command in order to get periodic triangular pulse

48. Write matlab command in order to get periodic rectagular pulse

49. Write matlab command in order to get periodic sawtooth waveform

50. Write matlab command in order to get sinc function

EXERCISE PROGRAM

1.Write a MATLAB program to find even part of a signal by considering 10 input samples.

2. Write a MATLAB program to find odd part of a signal by considering atleast  7 samples.

3. Write a MATLAB program to add even an odd part of a signal and see how matlab reacts for the above program.

4. Write a matlab program to get the out put as [-5, 3, 0, 8] as imaginary values and [2 4 6 8 0] as real values.

5. Write a MATLAB program to subtract even an odd part of a signal and see how matlab reacts  for the above program.

6. Write a MATLAB program to find real & imaginary  values of complex numbers[1+6i 2-5i 3 4+3i 5i].

7. Write a MATLAB program to find real & imaginary  values of complex numbers[2+6i 7+5i   3 9+3i 5i].

8. Write a MATLAB program to find real & imaginary  values of complex numbers[7+6i 2-5i 3 8+3i 7i].

9. Write a MATLAB program to find even & odd part of the sequence [5 6 3 2 3 4 7].

10. Write a MATLAB program to find even & odd part of the sequence [7 5 8 6 7].

11.Write a MATLAB program to find even part of a signal by considering 8 input samples.

12. Write a MATLAB program to find odd part of a signal by considering 10 samples.

13. Write a MATLAB program to multiply even and odd part of a signal and see how matlab reacts for the above program.

14. Write a matlab program to get the output as [2, 5, -1, 6] imaginary values and [4, 2, 8, 0] as real values.

15. Write a MATLAB program to divide even an odd part of a signal and see how matlab reacts for the above program.

16. Write a MATLAB program to find real & imaginary values of complex numbers [2+4i 1-6i 4+2i 8i].

17. Write a MATLAB program to find real & imaginary values of complex numbers [4+8i 6+4i   2 6+4i 6i].

18. Write a MATLAB program to find real & imaginary  values of complex numbers[4+4i 2-5i 4 6+4i 6i].

19. Write a MATLAB program to find even & odd part of the sequence [4 9 8 4 3 2 1].

20. Write a MATLAB program to find even & odd part of the sequence [5 7 6 7 8].

21. Write a MATLAB program to find even & odd part of the sequence [2 3 6 3 4 3 7].

22. Write a MATLAB program to find even & odd part of the sequence [5 4 6 8 6].

23.Write a MATLAB program to find even part of a signal by considering 6 input samples.

24. Write a MATLAB program to find odd part of a signal by considering 9 samples.

25. Write a matlab program to get the output as [2, 4, -6, 8] imaginary values and [1, 5, 9, 3] as real values.

Real Time Applications:

- Synthesis and manipulation: E.g. speech synthesis, music synthesis, graphics.

- Analysis: Seismic data, atmospheric data, stock market analysis.

EXPERMENT NO:5

LINEAR CONVOLUTION

AIM: -

To find the out put with linear convolution operation Using MATLAB Software.

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

Linear Convolution involves the following operations.

1. Folding

2. Multiplication

3. Addition

4. Shifting

These operations can be represented by a Mathematical Expression as follows:

x[n]= Input signal Samples

h[ n-k]= Impulse response co-efficient.

y[ n]= Convolution output.

n = No. of Input samples

h = No. of Impulse response co-efficient.

Example : X(n)={1 2 -1 0 1}, h(n)={ 1,2,3,-1}.

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

%Convolution of two signals%

clc;

clear all;

close all;

t=0:0.001:10;

x=sin(t);

h=square(t);

subplot(3,1,1);

plot(t,x,'g');

xlabel('time');

ylabel('amplitude');

title('sinusoidal signal');

subplot(3,1,2);

plot(t,h,'r');

xlabel('time');

ylabel('amplitude');

title('square function');

y=conv(x,h);

subplot(3,1,3);

plot(y);

xlabel('time');

ylabel('amplitude');

title('convolution signal');

%Convolution of two sequences%

clc;

clear all;

close all;

L=input('enter the length of 1st sequence');

M=input('enter the length of 2nd sequence');

x=input('enter the first sequence:x(n)=');

h=input('enter the second sequence:y(n)=');

N=0:(L+M-1);

y=conv(x,h);

subplot(3,1,1);

stem(x,'g');

xlabel('discrete time');

ylabel('x(n)');

title('1st sequence');

subplot(3,1,2);

stem(h,'r');

xlabel('discrete time');

ylabel('h(n)');

title('second sequence');

subplot(3,1,3);

stem(y);

xlabel('discrete time');

ylabel('y(n)');

title('convolution of two sequences');

OUTPUT:-

Convolution of two signals

Convolution of two sequences

enter the length of 1st sequence4

enter the length of 2nd sequence4

enter the first sequence:x(n)=[1 2 3 4]

enter the second sequence:y(n)=[1 2 3 4]

RESULT:-

In this experiment convolution of various signals have been

performed Using MATLAB.

VIVA QUESTIONS:-

1. Define Convolution?

2. Define Properties of Convolution?

3. What is the Difference Between Convolution& Correlation?

4. What are Dirchlet Conditions of Fourier Series?

5.What is Half Wave Symmetry?

6. Define Linear Convolution?

7. Define Properties of Convolution in time domain?

8. What is the Difference Between Linear Convolution& Circular Correlation?

9. What are Dirchlet Conditions for Fourier Transform?

10.What is Full Wave Symmetry?

11.What is Correlation?

12. What is the importance of correlation?

13. What is the difference b/w correlation and convolution?

14.What is Auto  Correlation?

15.What is Cross Correlation?

16.Relation between convolution and correlation?

17.State the commutative property of convolution?

18. State the associative property of convolution?

19. State the distributive property of convolution?

20.what is convolution integral?

21.What is quaterwave symmetry?

22.What is impulse response?

23. What is step response?

24 What is ramp response?

25. What is parabolic response?

26.How to calculate DC component of periodic signal?

27. How to calculate an component of periodic signal?

28. How to calculate bn component of periodic signal?

29. Relation between energy and correlation?

30.Defne amplitude and phase spectrum?

31.Explain hidden symmetry?

32.Explain Gibbs phenomenon?

33.What are the Dirichlet conditions?

34.Write the parsevals theorem?

35.What is rotation symmetry?

36.What is power spectrum?

37.What is compact form fourier series?

38.What is fourier transform of impulse signal?

39. What is fourier transform of step signal?

40. What is fourier transform of ramp signal?

41. What is fourier transform of signum function?

42. What is fourier transform of SINC function?

43.When a periodic is said to have a half wave symmetry?

44. When a periodic is said to have a quater wave symmetry?

45. When a periodic is said to have a odd symmetry?

46. When a periodic is said to have a even symmetry?

47.What is the relationship between cosine and trigonometric representation?

48.What is the condition for half wave symmetry?

49. What is the condition for quater wave symmetry?

50. What is the condition for odd symmetry?

EXERCISE PROGRAMS

1.Write the MATLAB program to perform convolution between the following sequences

X(n)=[1 -1 4 ], h(n) = [ -1 2 -3 1].

2. Write a mat lab program to perform the convolution between sinusoidal and ramp function and see how mat lab reacts to it.

3. Write a MATLAB program to perform convolution between square and step signal and see how mat lab reacts to it.

4. Write a MATLAB program to perform convolution between sinusoidal and ramp signal and see how mat lab reacts to it.

5. Write a MATLAB program to perform the convolution between X (n) = [1 2 3 5] and y (n) = [- 1 – 2] and see how matlab reacts to it.

6. Write a MATLAB program to perform the convolution between X (n) = [1 -3  5] and y (n) = [1 2 3 4] and see how matlab reacts to it.

7. Write a MATLAB program to perform the convolution between X (n) = [1 0 1 1] and y (n) = [1 0 0 0 0] and see how matlab reacts to it.

8.Write a MATLAB program to perform the convolution between X (n) = [1 1 1 1 0 0 0 0] and y (n) = [1 0 1 0 1 0 1 0] and see how matlab reacts to it.

9.Write a MATLAB program to perform the convolution between X (n) = [1 1 0 0] and y (n) = [1 1 1 1] and see how matlab reacts to it.

10.Write a MATLAB program to perform the convolution between X (n) = [6 7 8 9 10] and y (n) = [5 4 3 2 1] and see how matlab reacts to it.

11.Write the MATLAB program to perform convolution between the following sequences

X(n)=[2 -2 3 2, h(n) = [-2 4 -5 2].

12. Write a mat lab program to perform the convolution between step and ramp function and see how mat lab reacts to it.

13. Write a MATLAB program to perform convolution between square and step signal and see how mat lab reacts to it.

14. Write a MATLAB program to perform convolution between sinusoidal and square signal and see how mat lab reacts to it.

15. Write a MATLAB program to perform the convolution between X (n) = [2 3 4 5] and y (n) = [-2  –4] and see how matlab reacts to it.

16. Write a MATLAB program to perform the convolution between X (n) = [3 -4 6] and y (n) = [2 6 8 6] and see how matlab reacts to it.

17. Write a MATLAB program to perform the convolution between X (n) = [3 1 2 0] and y (n) = [2 4 3 4 5] and see how matlab reacts to it.

18.Write a MATLAB program to perform the convolution between X (n) = [2 2 1 0 2 1 1 0] and y (n) = [1 1 1 2 1 0 1 0] and see how matlab reacts to it.

19.Write a MATLAB program to perform the convolution between X (n) = [2 1 2 1] and y (n) = [1 2 1 2] and see how matlab reacts to it.

20.Write a MATLAB program to perform the convolution between X (n) = [5 6 9 8 7] and y (n) = [1 2 3 2 1] and see how matlab reacts to it.

21.Write the MATLAB program to perform convolution between the following sequences

X(n)=[4 -6 5 1], h(n) = [-1 3 -4 3].

22. Write a mat lab program to perform the convolution between ramp and step function and see how mat lab reacts to it.

23. Write a MATLAB program to perform convolution between square and step signal and see how mat lab reacts to it.

24. Write a MATLAB program to perform convolution between sinusoidal and step signal and see how mat lab reacts to it.

25. Write a MATLAB program to perform the convolution between X (n) = [1 5 6 5] and y (n) = [-3 –6] and see how matlab reacts to it.

Real Time Applications:

- The convolution is to determine the response y[n] of a system of a known impulse response h[n] for a given input signal x[n] to obtain y[n].

EXPERMENT NO: 6

AUTO CORRELATION AND CROSS CORRELATION BETWEEN

SIGNALS AND SEQUENCES

AIM: -

To compute auto correlation and cross correlation between signals and Sequences.

SOFTWARE REQURIED:-

1. MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

In Signal processing,When the autocorrelation function is normalized by mean and variance, it is sometimes referred to as the autocorrelation coefficient.Given a signal f(t), the continuous autocorrelationRff(T) is most often defined as the continuous cross-correlation integral of f(t)with itself, at lag T

### .

### The discrete autocorrelation Rat lag  j  for a discrete signal x(n) is

In signal processing, cross-correlation is a measure of similarity of two waveforms as a function of a time-lag applied to one of them. This is also known as a sliding dot product or sliding inner-product. It is commonly used for searching a long signal for a shorter, known feature. It has applications in pattern recognition, single particle analysis, electron tomographic averaging, cryptanalysis, and neurophysiology.

For continuous functions f and g, the cross-correlation is defined as:

where f* denotes the complex conjugate of f and t is the time lag.

Similarly, for discrete functions, the cross-correlation is defined as:

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

%Auto correlation for a signal%

clc;

clear all;

close all;

t=0:0.001:1;

x=cos(2*pi*3*t);

a=xcorr(x);

subplot(2,1,1);

plot(t,x,'g');

xlabel('time');

ylabel('amplitude');

title('input signal');

subplot(2,1,2);

plot(a,'r');

xlabel('time');

ylabel('amplitude');

title('auto correlation signal');

%cross correlation for a signal%

clc;

clear all;

close all;

t=0:0.001:1;

x=cos(2*pi*3*t);

y=cos(2*pi*5*t);

a=xcorr(x,y);

subplot(3,1,1);

plot(t,x,'g');

xlabel('time');

ylabel('amplitude');

title('1st signal');

subplot(3,1,2);

plot(t,y,'r');

xlabel('time');

ylabel('amplitude');

title('2nd signal');

subplot(3,1,3);

plot(a,'r');

xlabel('time');

ylabel('amplitude');

title('cross correlation signal');\

%Auto correlation for a sequence%

clc;

clear all;

close all;

x=input('enter sample values')

a=xcorr(x);

subplot(2,1,1);

stem(x,'g');

xlabel('time');

ylabel('amplitude');

title('input sequence');

subplot(2,1,2);

stem(a,'r');

xlabel('time');

ylabel('amplitude');

title('auto correlation sequence');

%cross correlation for asequence%

clc;

clear all;

close all;

x=input('enter first sequence');

y=input('enter second sequence');

a=xcorr(x,y);

subplot(3,1,1);

stem(x,'g');

xlabel('time');

ylabel('amplitude');

title('1st sequence');

subplot(3,1,2);

stem(y,'r');

xlabel('time');

ylabel('amplitude');

title('2nd sequence');

subplot(3,1,3);

stem(a,'r');

xlabel('time');

ylabel('amplitude');

title('cross correlation sequence');

OUTPUT:-

Auto correlation for a signal

cross correlation for a signal

Auto correlation for a sequence

enter sample values[1 2 3 4]

x =

1     2     3     4

cross correlation for asequence

enter first sequence[1 2 3 4]

enter second sequence[1 2 3 4]

RESULT:-

In this experiment correlation of various signals have been performed Using

MATLAB.

VIVA QUESTIONS:-

1.MemorizeCorrelation?

2. Define Auto-Correlation?

3. Define Cross-Correlation?

4. What is the importance of correlation?

5.Differenciate b/w correlation and convolution?

6. Explain two Properties of Auto-Correlation?

7. Explain two Properties of Cross-Correlation?

8. Relate difference b/w Auto-correlation and Cross-Correlation?

9. Define ESD?

10.State PSD?

11.What is Correlation?

12.What is the importance of correlation?

13. Compare difference b/w correlation and convolution?

14.What is Auto  Correlation?

15.State is Cross Correlation?

16. Locate Simulation?

17.List need For Simulation?

18. Classification Of Simulation Systems?

19. Explain Discrete-event Simulation?

20.  List Some Open Source Circuit Simulator Software List?

21.  What are The Abstraction Levels For Circuit Simulation?

22. What are Circuit-level Simulators?

23. What are Logical Level Simulators?

24.State the Commutative Properties of Convolution?

25.What are the Steps involved in Calculating Convolution Sum?

26.State he Significance of Difference Equations?

27.Write The Difference Equation For Discrete Time System?

28.List out  the Properties of Convolution?

29.State the Commutative Properties of Convolution?

30.State the Associative Properties of Convolution?

31.Memorize Causal LTI DT System?

32.What are the properties of continous Time fourier series.

33.State laplace transform.

34.Report the condition for convergance of L.T?

35. State region of convergance ?

36.State shifting property of L.T?

37.What is normalized correlation.

38.Differentiate cross correlation nad Auto correlation.

39.Justify  need For Simulation.

40. What are Circuit-level Simulators?

41. How to determine Transpose of Matrix?

42.How to determine  Trace of Matrix?

43..How to determine Inverse of Matrix?

44. Define Periodic and a periodic Signal?

45.Write the relation between Step and Impulse signals?

46. Write the relation between Ramp and impulse signals?

48.What is System?

49.Explain Even Signals?

50. Memorize Causal LTI DT System?

EXERCISE PROGRAMS

1.Write a MATLAB  program to compute auto correlation between signals and Sequences. x=cos(2*pi*10*t),y=cos(2*pi*15*t).

2. Write a MATLAB  program to compute cross correlation between signals and Sequences. x=cos(2*pi*7*t),y=cos(2*pi*14*t).

3. Write  a MATLAB program to compute the cross correleation  between signals and Sequences. x=cos(2*pi*10*t),y=cos(2*pi*15*t)  by increasing the  amplitude of the signal by 3 times and verify how matlab reacts to it.

4. Write  a MATLAB program to compute the auto correleation  between signals and Sequences. x=cos(2*pi*15*t),y=cos(2*pi*10*t) by increasing the  amplitude of the signal by 2 times and verify how matlab reacts to it.

5. Write a MATLAB program to compute auto correlation between x  =sin(2*pi*5*t) ,  y =  sin(2*pi*10*t). and see how matlab reacts to it.

6. Write a MATLAB program to compute cross correlation between x  =sin(2*pi*5*t) ,  y =  cos(2*pi*10*t). and see how matlab reacts to it.

7.Write a MATLAB  program to compute cross correlation between two signals x=9*cos(2*pi*7*t),y=5*cos(2*pi*14*t).

8. Write a MATLAB  program to compute cross correlation between two Sequences [1 2 3 4 5]&[9 8 7 6 5].

9. Write a MATLAB  program to compute auto correlation between two signals x=9*cos(2*pi*7*t),y=5*cos(2*pi*14*t).

10. Write a MATLAB  program to compute auto correlation between two Sequences [2 4 3 4 5] & [7 9 4 6 3].

11.Write a MATLAB  program to compute auto correlation between signals and Sequences. x=cos(2*pi*20*t),y=cos(2*pi*10*t).

12. Write a MATLAB  program to compute cross correlation between signals and Sequences. x=cos(4*pi*8*t),y=cos(4*pi*10*t).

13. Write a MATLAB program to compute the cross correleation  between signals and Sequences. x=cos(2*pi*20*t),y=cos(2*pi*10*t)  by increasing the  amplitude of the signal by 2 times and verify how matlab reacts to it.

14. Write  a MATLAB program to compute the auto correleation  between signals and Sequences. x=cos(2*pi*5*t),y=cos(2*pi*20*t) by increasing the  amplitude of the signal by 4 times and verify how matlab reacts to it.

15. Write a MATLAB program to compute auto correlation between x =sin(2*pi*5*t) ,  y =  sin(4*pi*20*t). and see how matlab reacts to it.

16. Write a MATLAB program to compute cross correlation between x = sin(3*pi*5*t) ,  y =  cos(4*pi*5*t). and see how matlab reacts to it.

17.Write a MATLAB  program to compute cross correlation between two signals x=4*cos(2*pi*5*t),y=6*cos(2*pi*10*t).

18. Write a MATLAB  program to compute cross correlation between two Sequences [2 6 4 8 5] & [8 9 6 7 5].

19. Write a MATLAB  program to compute auto correlation between two signals x=5*cos(2*pi*10*t),y=5*cos(2*pi*10*t).

20. Write a MATLAB  program to compute auto correlation between two Sequences [1 1 4 4 5] & [9 9 7 5 5].

21.Write a MATLAB  program to compute auto correlation between signals and Sequences. x=cos(3*pi*10*t),y=cos(4*pi*5*t).

22. Write a MATLAB  program to compute cross correlation between signals and Sequences. x=cos(6*pi*6*t),y=cos(4*pi*5*t).

23. Write a MATLAB program to compute the cross correleation  between signals and Sequences. x=cos(2*pi*10*t),y=cos(2*pi*5*t)  by increasing the  amplitude of the signal by 3 times and verify how matlab reacts to it.

24. Write  a MATLAB program to compute the auto correleation  between signals and Sequences. x=cos(4*pi*5*t),y=cos(2*pi*5*t) by increasing the  amplitude of the signal by 4 times and verify how matlab reacts to it.

25. Write a MATLAB program to compute auto correlation between x =sin(4*pi*5*t) ,  y =  sin(4*pi*10*t). and see how matlab reacts to it.

Real Time Applications:

- Correlation is used to extract second (and higher) order statistics from any random signal.

EXPERMENT NO:7

VERIFICATION OF LINEARITY AND TIME INVARIANCE

PROPERTIES OF A GIVEN CONTINUOUS /DISCRETE SYSTEM.

AIM: -

To compute linearity and time invariance properties of a given continuous /discrete

System.

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

LINEARITY PROPERTY

LINEAR TIME INVARIENT SYSTEMS(LTI)

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

%Program1:%

clc;

clear all;

close all;

n=0:40; a=2; b=1;

x1=cos(2*pi*0.1*n);

x2=cos(2*pi*0.4*n);

x=a*x1+b*x2;

y=n.*x;

y1=n.*x1;

y2=n.*x2;

yt=a*y1+b*y2;

d=y-yt;

d=round(d)

if d

disp('Given system is not satisfy linearity property');

else

disp('Given system is satisfy linearity property');

end

subplot(3,1,1), stem(n,y);

grid;

subplot(3,1,2), stem(n,yt);

grid;

subplot(3,1,3), stem(n,d);

grid;

%Program2:%

clc;

clear all;

close all;

n=0:40; a=2; b=-3;

x1=cos(2*pi*0.1*n);

x2=cos(2*pi*0.4*n);

x=a*x1+b*x2;

y=x.^2;

y1=x1.^2;

y2=x2.^2;

yt=a*y1+b*y2;

d=y-yt;

d=round(d);

if d

disp('Given system is not satisfy linearity property');

else

disp('Given system is satisfy linearity property');

end

subplot(3,1,1), stem(n,y);

grid;

subplot(3,1,2), stem(n,yt);

grid;

subplot(3,1,3), stem(n,d);

grid;

%Program3:%

clc;

close all;

clear all;

n=0:40;

D=10;

x=3*cos(2*pi*0.1*n)-2*cos(2*pi*0.4*n);

xd=[zeros(1,D) x];

y=n.*xd(n+D);

n1=n+D;

yd=n1.*x;

d=y-yd;

if d

disp('Given system is not satisfy time shifting property');

else

disp('Given system is satisfy time shifting property');

end

subplot(3,1,1),stem(y),grid;

subplot(3,1,2),stem(yd),grid;

subplot(3,1,3),stem(d),grid;

OUTPUT:-

Program1:

Program2:

Program3:

VIVA:

1. Define Systems?

2. What is LTI Systems?

3. Describe LTV Systems?

4.Summarize importance of Linear Systems?

5 Differentiate b/w Linear and Non-Linear Systems?

6. Summarize Properties of LTI Systems?

7. Explain two Properties of LTV Systems?

8. Appraise Superposition Principal?

9.  Compare between CT and DT systems?

10.Memorize Causality of LTI Systems?

11.State  Properties Of Convolution?

12.List the Applications Of Correlation?

13.Locate  DFT ?

14.List classification of Continuous Time Signals? Name Them?

15Revise Examples of Causal Signal?

16.Apraise Amplitude Scaling And Time Scaling?

17.Classify Discrete Time Signal?

18.What is meant by Step Response Of The Dt System?

19.Define Impulse Response of a Dt System?

20.State The Significance of Difference Equations?

21.Write The Difference Equation For Discrete Time System?

22.What are the Properties of Convolution?

23.State the Commutative Properties of Convolution?

24.State the Associative Properties of Convolution?

25.Memorize Causal Lti Dt System?

26.What are the properties of continous Time fourier series.

27.State laplace transform.

28.Report the condition for convergance of L.T?

29. State region of convergance ?

30.State shifting property of L.T?

31.Memorize transfer function?

32.Memorize convolution property of L.T?

33. Define Time Variant And Time Invariant System?

34. State The Methods For Evaluating Inverse Z-transform.?

35. How to obtain The output Sequence of Linear Convolution through Circular Convolution?

36. What is Zero Padding?what Are Its Uses?

37. Define Sectional Convolution?

38. Why FFT Is Needed?

39. Distinguish Between Linear Convolution And Circular Convolution Of Two Sequences?

40. Justify Differences And Similarities Between Dif And Dit Algorithms?

41. What are The Different Types Of Filters Based On Impulse Response?

42. What are The Different Types Of Filters Based On Frequency Response?

43.What is Power signals?

44.Define Systems?

45.How to determine time period of Periodic signals?

46.Define Frequency of signals?

47.What is the difference b/w stem&plot?

48.What is Energy Signals?

49.What is Power signals?

50.Define Systems?

EXSERCISE QUESTIONS

- Write a MATLAB program to verify the linearity prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the linearity property or not.

- Write a MATLAB program to verify the linearity prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the linearity property or not

- Write a MATLAB program to verify the linearity prpoerty of the following sequency   x1= sin(2*pi*0.1*n); cos(2*pi*0.3*n), and chech whether it satisfies the linearity property or not

- Write a MATLAB program to verify the time invariance prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the time invariance property or not.

- Write a MATLAB program to verify the time invariance prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the time invariance property or not

- Write a MATLAB program to verify the  time invariance prpoerty of the following sequency   x1= sin(2*pi*0.1*n); cos(2*pi*0.3*n), and chech whether it satisfies the time invariance property or not.

- Write a MATLAB program to verify the  time variance prpoerty of the following sequency   x1= sin(2*pi*0.1*n); cos(2*pi*0.3*n), and chech whether it satisfies the time invariance property or not.

- Write a MATLAB program to verify the non linearity prpoerty of the following sequency   x1= sin(2*pi*0.1*n); cos(2*pi*0.3*n), and chech whether it satisfies the linearity property or not

- Write a MATLAB program to verify the time variance prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the time invariance property or not.

- Write a MATLAB program to verify the non linearity prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the linearity property or not.

- Write a MATLAB program to verify the linearity prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the linearity property or not.

- Write a MATLAB program to verify the linearity prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the linearity property or not

- Write a MATLAB program to verify the linearity prpoerty of the following sequency   x1= sin(2*pi*0.1*n); cos(2*pi*0.3*n), and chech whether it satisfies the linearity property or not

- Write a MATLAB program to verify the time invariance prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the time invariance property or not.

- Write a MATLAB program to verify the time invariance prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the time invariance property or not

- Write a MATLAB program to verify the  time invariance prpoerty of the following sequency   x1= sin(2*pi*0.1*n); cos(2*pi*0.3*n), and chech whether it satisfies the time invariance property or not.

- Write a MATLAB program to verify the  time variance prpoerty of the following sequency   x1= sin(2*pi*0.1*n); cos(2*pi*0.3*n), and chech whether it satisfies the time invariance property or not.

- Write a MATLAB program to verify the non linearity prpoerty of the following sequency   x1= sin(2*pi*0.1*n); cos(2*pi*0.3*n), and chech whether it satisfies the linearity property or not

- Write a MATLAB program to verify the time variance prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the time invariance property or not.

- Write a MATLAB program to verify the non linearity prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the linearity property or not.

- Write a MATLAB program to verify the linearity prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the linearity property or not.

- Write a MATLAB program to verify the linearity prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the linearity property or not

- Write a MATLAB program to verify the linearity prpoerty of the following sequency   x1= sin(2*pi*0.1*n); cos(2*pi*0.3*n), and chech whether it satisfies the linearity property or not

- Write a MATLAB program to verify the time invariance prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the time invariance property or not.

- Write a MATLAB program to verify the time invariance prpoerty of the following sequency   x1= sin(2*pi*1*n); x2= sin(2*pi*2*n), and chech whether it satisfies the time invariance property or not

Real Time Applications:

- Wave Propagation such as sound and electromagnetic waves.

- Electrical circuits composed of resistors, capacitors and inductors.

EXPERMENT NO:8

COMPUTATION OF UNIT SAMPLE, UNIT STEP AND

SINUSOIDAL RESPONSE OF THE GIVEN LTI SYSTEM AND

VERIFYING ITS PHYSICAL REALIZABILITY AND STABILITY

PROPERTIES.

AIM: -

To Unit Step And Sinusoidal Response Of The Given LTI System And Verifying

Its Physical Realizability And Stability Properties.

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

A discrete time system performs an operation on an input signal based on predefined

criteria to produce a modified output signal. The input signal x(n) is the system

excitation, and y(n) is the system response. The transform operation is shown as,

If the input to the system is unit impulse i.e. x(n) = δ(n) then the output of the system is

known as impulse response denoted by h(n) where,

h(n) = T[δ(n)]

we know that any arbitrary sequence x(n) can be represented as a weighted sum of

discrete impulses. Now the system response is given by,

For linear system (1) reduces to

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

%calculate and plot the impulse response and step response%

clc;

clear all;

close all;

b=[1];

a=[1,-1,.9];

x=impseq(0,-20,120);

n = [-20:120];

h=filter(b,a,x);

subplot(3,1,1);stem(n,h);

title('impulse response');

xlabel('n');ylabel('h(n)');

=stepseq(0,-20,120);

s=filter(b,a,x);

s=filter(b,a,x);

subplot(3,1,2);

stem(n,s);

title('step response');

xlabel('n');ylabel('s(n)')

t=0:0.1:2*pi;

x1=sin(t);

%impseq(0,-20,120);

n = [-20:120];

h=filter(b,a,x1);

subplot(3,1,3);stem(h);

title('sin response');

xlabel('n');ylabel('h(n)');

figure;

zplane(b,a);

OUTPUT:-

plot the impulse response and step response

RESULT:-

In this experiment computation of unit sample, unit step and

sinusoidal response of the given lti system and verifying its physical realizability

andstability properties Using MATLAB.

VIVA QUESTIONS:-

1.List out operations that can be performed on signals and sequence?

2.State causality?

3.Define scaling property and give its importance?

4.Judge shifting  property and give its importance?

5.Memorize folding property and give its importance?

6.Define  stability?

7.Evaluate the importance of Causality?

8.Give the importance of Stability?

9.Define Impulse Response?

10.Locate Step Response?

11.What is meant by Step Response of the DT System?

12.Define Impulse Response of a DT System?

13.State the Significance of Difference Equations?

14.Write the Difference Equation For Discrete Time System?

15.What are the Properties of Convolution?

16.Memorize the Commutative Properties of Convolution?

17.State the Associative Properties of Convolution?

18.Locate Causal LTI DT System?

19. Define Causal LTI DT System?

20.Memorize stability?

21.State Reliability?

22.Relate stability nad realiabilty of a signal?

23. compare stability nad realiabilty of a sequence?

24. What are the classification of the System based on Unit Sample Response?

25. Memorize is Recursive System?

26. State is Non-Recursive System?

27. What Is Natural Response?

28. Locate Is Zero Input Response?

29. Define Is Forced Response?

30. Examine  Complete Response?

31. Relation between ramp and parabolic signal?

32.Define orthogonality between signals?

33. State properties of convolution?

34.Give some examples of causal system?

35. State superposition theorem?

36.How the DT system is represented?

37.What is folding operation?

38.State  is shifting operation?

39.Locateis time scaling operation?

40. Define is SINC function?

41. What is amplitude scaling operation?

42.Memorize sampling function?

43. justify  summation operation?

44. What is multiplication operation?

45. List some examples of random signal?

46. Relate stability nad realiabilty of a signal?

47.  Justify of feedback system?

48.Memorize conditions for distortionless transmission system?

49.Formulate expression for rectangular function?

50. Write expression for tringular function?

EXERCISE PROGRAM:-

1. Write a MATLAB program for generating u(n)-u(n-1).

2. Write a MATLAB program for generating delayed unit step response

3. Write a MATLAB program for generating delayed impulse response

4. Write a MATLAB program for generating u(n)+u(n-1) and verify how matlab reacts to it.

5. write a MATLAB program to find the energy & power of the signal x(t)=10sin(10t).

6. write a MATLAB program to find the even & odd of the signal x(t)=e^2t.

7. write a MATLAB program to find the even & odd of the signal x(t)=sin(t).

8. write a MATLAB program to find the even & odd of the signal x(t)=cos(t).

9. write a MATLAB program to find the energy & power of the signal x(t)=sin(t).

10. write a MATLAB program to find the energy & power of the signal x(t)=e^2t.

11. Write a MATLAB program for generating u(n-2)-u(n-1).

12. Write a MATLAB program for generating delayed step response with magnitude 2

13. Write a MATLAB program for generating delayed impulse response with magnitude 2.

14. Write a MATLAB program for generating u(n+1)+u(n-1) and verify how matlab reacts to it.

15. Write a MATLAB program to find the energy & power of the signal x(t)=20sin(5t).

16. Write a MATLAB program to find the even & odd of the signal x(t)=e^3t.

17. Write a MATLAB program to find the even & odd of the signal x(t)=sin(t)+cos(t).

18. Write a MATLAB program to find the even & odd of the signal x(t)=cos(t)-sin(t).

19. Write a MATLAB program to find the energy & power of the signal x(t)=sin(t)-cos(t).

20. Write a MATLAB program to find the energy & power of the signal x(t)=e^4t.

21. Write a MATLAB program for generating u(n+2)-u(n-1).

22. Write a MATLAB program for generating delayed unit step response

23. Write a MATLAB program for generating delayed impulse response

24. Write a MATLAB program for generating u(n-3)+u(n-1) and verify how matlab reacts to it.

25. Write a MATLAB program to find the energy & power of the signal x(t)=5sin(20t).

Real Time Applications:

- Electronic Circuits such as amplifiers and filters.

- Mechanical motion from the interaction of masses, springs and dashpots (dampers).

EXPERMENT NO:9

DEMONSTRATION OF GIBBS PHENOMENON

AIM: -

To demonstrate Gibbs Phenomenon using MATLAB.

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

Gibbs Phenomenon:

The peculiar manner in which the Fourier series of a piecewise continuously differentiable

periodic function behaves at a jump discontinuity: the nth partial sum of the Fourier series

has large oscillations near the jump, which might increase the maximum of the partial sum

above that of the function itself. The overshoot does not die out as the frequency increases,

but approaches a finite limit.

The Gibbs phenomenon involves both the fact that Fourier sums overshoot at a jump

discontinuity, and that this overshoot does not die out as the frequency increases.

The best known version of the Gibbs phenomenon is the overshoot that arises when a discontinuous

function is represented by a truncated set of Fourier expansion terms. The situation is similar if the

truncated Fourier expansion is instead obtained by means of interpolation on an equispaced grid.

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

%Gibbs phenomenon%

clc;

clear all;

close all;

t=0:0.01:1;

N=input('enter no of harmonics');

x=square(2*pi*t);

plot(t,x);

hold on;

gp=0;

for n=1:2:N;

gp=gp+(4/(n*pi))*sin(2*pi*n*t);

end;

plot(t,gp);

hold off;

xlabel('time');

ylabel('amplitude');

title('gibbs phenomenon');

OUTPUT:-

Gibbs phenomenon

enter no of harmonics5

RESULT:-

In this experiment Gibbs phenomenon have been demonstrated

Using MATLAB.

EXERCISE PROGRAM:-

1.Write the MATLAB program to perform convolution between the following sequences

X(n)=[1 8 9 3 ], h(n) = [ 2 4 5 9].

2. Write a mat lab program to perform the convolution between sinusoidal and ramp function and see how mat lab reacts to it.

3. Write a MATLAB program to perform convolution between square and step signal and see how mat lab reacts to it.

4. Write a MATLAB program to perform convolution between sinusoidal and ramp signal and see how mat lab reacts to it.

5. Write a MATLAB program to perform the convolution between X (n) = [8 5 6 9] and y (n) = [- 1 – 2] and see how matlab reacts to it.

6. Write a MATLAB program to perform the convolution between X (n) = [1 -3 4 5] and y (n) = [1 2 3 4] and see how matlab reacts to it.

7. Write a MATLAB program to perform the convolution between X (n) = [1 0 1 1] and y (n) = [1 0 0 0 0] and see how matlab reacts to it.

8.Write a MATLAB program to perform the convolution between X (n) = [1 1 1 1 0 0 0 0] and y (n) = [1 0 1 0 1 0 1 0] and see how matlab reacts to it.

9.Write a MATLAB program to perform the convolution between X (n) = [1 1 0 0] and y (n) = [1 1 1 1] and see how matlab reacts to it.

10.Write a MATLAB program to perform the convolution between X (n) = [6 9 7 5 2] and y (n) = [8 5 6 4 2] and see how matlab reacts to it.

11.Write the MATLAB program to perform convolution between the following sequences

X(n)=[2 4 9 3], h(n) = [1 3 7 5].

12. Write a mat lab program to perform the convolution between step and ramp function and see how mat lab reacts to it.

13. Write a MATLAB program to perform convolution between sinusoidal and step signal and see how mat lab reacts to it.

14. Write a MATLAB program to perform convolution between impulse and ramp signal and see how mat lab reacts to it.

15. Write a MATLAB program to perform the convolution between X (n) = [3 2 6 9] and y (n) = [-2 –4] and see how matlab reacts to it.

16. Write a MATLAB program to perform the convolution between X (n) = [2 -4 2 5] and y (n) = [4 2 3 4] and see how matlab reacts to it.

17. Write a MATLAB program to perform the convolution between X (n) = [2 1 0 1] and y (n) = [1 0 1 1] and see how matlab reacts to it.

18. Write a MATLAB program to perform the convolution between X (n) = [1 2 1 2 0 1 0 1] and y (n) = [2 0 1 1 0 0 2 0] and see how matlab reacts to it.

19.Write a MATLAB program to perform the convolution between X (n) = [2 2 0 2] and y (n) = [1 1 2 2] and see how matlab reacts to it.

20. Write a MATLAB program to perform the convolution between X (n) = [2 1 4 5 3] and y (n) = [2 1 6 2 1] and see how matlab reacts to it.

21.Write the MATLAB program to perform convolution between the following sequences

X(n)=[2 8 7 3], h(n) = [1 4 2 9].

22. Write a mat lab program to perform the convolution between sinusoidal and ramp function and see how mat lab reacts to it.

23. Write a MATLAB program to perform convolution between square and step signal and see how mat lab reacts to it.

24. Write a MATLAB program to perform convolution between sinusoidal and ramp signal and see how mat lab reacts to it.

25. Write a MATLAB program to perform the convolution between X (n) = [4 5 2 1] and y (n) = [-2 – 1] and see how matlab reacts to it.

VIVA QUESTIONS:-

1. Define Gibb’s Phenomenon?

2. What is the importance of Gibb’s Phenomenon?

3. What is Static and Dynamic System?

4. What is Causal Signal?

5. What is Causality Condition of the Signal?

6. Define Noise?

7. What is the importance of FS?

8. What is the importance of FT?

9. Define LTV system?

10.Define LTI system?

11.State the Commutative Properties of Convolution?

12..State the Associative Properties of Convolution?

13.Memorize Causal Lti Dt System?

14.What are the properties of continous Time fourier series.

15.State laplace transform.

16.Report the condition for convergance of L.T?

17. State region of convergance ?

18.State shifting property of L.T?

19.Memorize transfer function?

20.Memorize convolution property of L.T?

21. What Are The Desirable Characteristics Of The Window Function?

23. What Are The Advantages Of Kaiser Window?

24. For What Type Of Filters Frequency Sampling Method Is Suitable?

25.State Equations Used to Convert the Fir Filter Coefficients to Lattice Filter Coefficient.

26. How One Can Design Digital Filters From Analog Filters?

27. Give The Bilinear Transform Equation Between S-plane And Z-plane?

28. What Is Bilinear Transformation?

29. What Is The Advantage Of Cascade Realization?

30. What Is Meant By Sign Magnitude Representation?

31.Define Symetric and Anti-Symmetric Signals?

32.Define Continuous and Discrete Time Signals?

33.What are the Different types of representation of discrete time signals?

34.What are the Different types of Operation performed on signals?

35.State Parseval’s Power theorem for a periodic signal?

36. What is Cross-correlation ?

37.Define ESD? Explain the relation between ESD and Auto-correlation ?

38.Define PSD? Explain the relation between PSD and Auto-correlation ?

39. State is Non-Recursive System?

40.What Is Natural Response?

41 Locate Is Zero Input Response?

42. Define Is Forced Response?

43. Examine  Complete Response?

44. Relation between ramp and parabolic signal?

45.Define orthogonality between signals?

46. State properties of convolution?

47.Give some examples of causal system?

48. State superposition theorem?

49.What is the importance of FT?

50. Define LTV system?

Real Time Applications:

- The Gibbs phenomenon, discovered by Henry Wilbraham and rediscovered by J. Willard Gibbs (1899), is the peculiar manner in which the Fourier series of a piecewise continuously differentiable periodic function behaves at a jump discontinuity.

EXPERMENT NO: 10

FINDING THE FOURIER TRANSFORM OF A GIVEN SIGNAL

AND PLOTTING ITS MAGNITUDE AND PHASE SPECTRUM

AIM: -

To obtain Fourier Transform and Inverse Fourier Transform of a given

signal / sequence and to plot its Magnitude and Phase Spectra.

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

Fourier Transform Theorems:

We may use Fourier series to motivate the Fourier transform as follows. Suppose that ƒ is

a function which is zero outside of some interval [−L/2, L/2]. Then for any T ≥ L we may

expand ƒ in a Fourier series on the interval [−T/2,T/2], where the "amount" of the wave

e2πinx/T in the Fourier series of ƒ is given by

By definition

The fast Fourier transform (FFT) is an efficient algorithm for computing the DFT of a

sequence; it is not a separate transform. It is particularly useful in areas such as signal and

image processing, where its uses range from filtering, convolution, and frequency analysis to

power spectrum estimation

For length N input vector x, the DFT is a length N vector X, with elements

N

X(k) = sum x(n)*exp(-j*2*pi*(k-1)*(n-1)/N), 1 <= k <= N.

n=1

The inverse DFT (computed by IFFT) is given by

N

x(n) = (1/N) sum X(k)*exp( j*2*pi*(k-1)*(n-1)/N), 1 <= n <= N.

k=1

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

%Fourier Transform%

clc

clear all;

close all;

syms t;

x=exp(-2*t)*heaviside(t);

y=fourier(x);

disp('Fourier Transform of input signal');

y

z=ifourier(y);

disp('Inverse Fourier Transform of input signal');

z

mg=abs(y);

subplot(2,1,1);

ezplot(mg);

xlabel('time');

ylabel('amplitude');

title('magnitude spectrum of a input signal');

grid;

pha=atan(imag(y)/real(y));

subplot(2,1,2);

ezplot(pha);

xlabel('time');

ylabel('amplitude');

title('phasespectrum of a input signal');

grid;

OUTPUT:-

Fourier Transform

Fourier Transform of input signal

Y=

1/(2+w*i)

Inverse Fourier Transform of input signal

Z=

Heaviside(x)/exp(2*x)

RESULT:-

In this experiment the fourier transform of a given signal and plotting

its magnitude and phase spectrum have been demonstrated using MATLAB.

VIVA QUESTIONS:-

1.Define Fourier Series?

2.What are the properties of Continuous-Time Fourier Series?

3. What is the Sufficient condition for the existence of F.T?

4. Define the F.T of a signal?

5. What is the difference b/w F.T&F.S?

6.Applications of Fourier transform?

7.what is plot?

8.what is stem?

9.what is difference between plot and stem?

10.what is fourier transform?

11.what is subplot?

12.what is formula of  F.T?

13.what is sin signal?

14.what is cos signal?

15. Define the F.T of a signal?

16. What is the difference b/w F.T&F.S?

17.Applications of Fourier transform?

18.What is orthogonal signal?

19.what is system bandwidth?

20.what is signal bandwidth?

21.Define the F.T of a signal?

22. What is the difference b/w F.T&T.F?

23.what is fourier transform?

24.what is subplot?

25.what is formula of  F.T?

26.what is sin signal?

27.what is cos signal?

28. Define the F.T of a signal?

29. What is the difference b/w F.T&F.S?

30.Applications of Fourier transform?

31.What is orthogonal signal?

32.Applications of Fourier transform?

33.Applications of Fourier series?

34.What is orthogonal signal?

35.what is system bandwidth?

36.what is signal bandwidth?

37.Define the F.T of a signal?

38. What is the difference b/w F.T&T.F?

39.what is fourier transform?

40.what is lineat system?

41.what is non linear system

42.what is even signal

43.what is odd signal

44.what is command for foldind signal?

45.what is clc?

46.applications of BS LAB?

47.what is causal system

48.what is non causal system ?

49.what is LTI system?

50.what is system?

EXERCISE PROGRAMS

1.Write a MATLAB program to find the cross correlation using FFT.

2. Write a MATLAB program to find F.T of the causal signal x(t)=t

3. Write a MATLAB program to find F.T of the causal signal x(t)=e^-at

4. Write a MATLAB program to find F.T of the causal signal x(t)=e^at

5. Write a MATLAB program to find F.T of the causal signal x(t)=te^at

6. Write a MATLAB program to find F.T of the causal signal x(t)=te^-at

7. Write a MATLAB program to find F.T of the causal signal x(t)=cost

8. Write a MATLAB program to find F.T of the causal signal x(t)=sint

9. Write a MATLAB program to find F.T of the causal signal x(t)=e^atcos(t)

10. Write a MATLAB program to find F.T of the causal signal x(t)= e^atsin(t)

11.Write a MATLAB program to find the cross correlation using DFT.

12. Write a MATLAB program to find F.T of the causal signal x(t)=at

13. Write a MATLAB program to find F.T of the causal signal x(t)=10e^-at

14. Write a MATLAB program to find F.T of the  signal x(t)=1

15. Write a MATLAB program to find F.T of the causal signal x(t)=impulse function

16. Write a MATLAB program to find F.T of the causal signal x(t)=Delta function

17. Write a MATLAB program to find F.T of the causal signal x(t)=cos(at)

18. Write a MATLAB program to find F.T of the causal signal x(t)=sin(at)

19. Write a MATLAB program to find F.T of the causal signal x(t)=e^atcos(at)

20. Write a MATLAB program to find F.T of the causal signal x(t)= e^atsin(at)

21. .Write a MATLAB program to find the auto correlation using FFT.

22. Write a MATLAB program to find F.T of the causal signal x(t)=5t

23. Write a MATLAB program to find F.T of the causal signal x(t)=5e^-at

24. Write a MATLAB program to find F.T of the causal signal x(t)=5e^at

25. Write a MATLAB program to find F.T of the causal signal x(t)=5te^at

Real Time Applications:

Fourier transforms having following applications

- Analysis of differential equations

- Fourier transform spectroscopy

- Quantum mechanics

- Signal processing applications etc

.

EXPERMENT NO:11

LAPLACE TRANSFORM

AIM: -

To perform waveform synthesis using Laplace Transforms of a given signal

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

Bilateral Laplace transform :

When one says "the Laplace transform" without qualification, the unilateral or one-sided

transform is normally intended. The Laplace transform can be alternatively defined as the

bilateral Laplace transform or two-sided Laplace transform by extending the limits of

integration to be the entire real axis. If that is done the common unilateral transform

simply becomes a special case of the bilateral transform where the definition of the

function being transformed is multiplied by the Heaviside step function.

The bilateral Laplace transform is defined as follows:

Inverse Laplace transform

The inverse Laplace transform is given by the following complex integral, which is

known by various names (the Bromwich integral, the Fourier-Mellin integral, and

Mellin's inverse formula):

Example :

Let y(t)=exp(t). We have

The integral converges if s>1. The functions exp(t) and 1/(s-1) are partner functions.

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

%Laplace Transform%

clc

clear all;

close all;

syms t;

x=exp(-2*t)*heaviside(t);

y=laplace(x);

disp('Laplace Transform of input signal');

y

z=ilaplace(y);

disp('Inverse Laplace Transform of input signal');

z

OUTPUT:-

Laplace Transform

Laplace Transform of input signal

Y=

1/(s+2)

Inverse Laplace Transform of input signal

Z=

1/exp(2*t)

RESULT:-

Laplace and inverse laplace transforms were verified using MATLAB functions.

VIVA QUESTIONS:-

1.define laplace-transform?

2. What is the condition for convergence of the l.t?

3. What is the region of convergence(roc)?

4. State the shifting property of l.t?

5. State convolution property of l.t?

6.define i.l.t?

7.applications of lti system

8.what is system?

9.what is signal?

10.what is fiurier transorm?

11.what is fourier series?

12.what is l.t.i property?

13.write one property of lti system?

14.whta is the command fliplr?>

15what is system?

16.what is orthogonal function?

17.what is tansfer function?

18.what is system bandwidth?

19.wghat is signal bandwidth?

20.what is fouries series?

21.what is convolution?

22.what is additive property?

23.what is cumilative property?

24.what are the types of signals?

25. What is ortogonal function?

26.what is co efficients?

27.what is fourier series?

28.what is fourier transform?

29.what is f.s?

30.what is convolutiojn?

31.what is fourier transorm?

32.what is system?

33.what is signal?

34.what is trignometric series?

35.application of convolution?

36.applications of lti system?

37.what is stable system?

38.what is system?

39.what is signal?

40.what is fiurier transorm?

41.what is covolution property?

42.what is auto correlation?

43.what is auto correlation?

44.what is correlation?

45.what is convolution sum?

46.what is filter?

47.what is ideal filter?

48.what is ideal bpf?

49.what is ideal lpf?

50.Application of bandpass filter?

exercise programs

1.Write a MATLAB program to find L.T of the causal signal x(t)=t^2

2. Write a MATLAB program to find L.T of the causal signal x(t)=t

3. Write a MATLAB program to find L.T of the causal signal x(t)=e^-at

4. Write a MATLAB program to find L.T of the causal signal x(t)=e^at

5. Write a MATLAB program to find L.T of the causal signal x(t)=te^at

6. Write a MATLAB program to find L.T of the causal signal x(t)=te^-at

7. Write a MATLAB program to find L.T of the causal signal x(t)=cost

8. Write a MATLAB program to find L.T of the causal signal x(t)=sint

9. Write a MATLAB program to find L.T of the causal signal x(t)=e^atcost

10. Write a MATLAB program to find L.T of the causal signalv x(t)= e^atsint.

11.Write a MATLAB program to find the cross correlation using DFT.

12. Write a MATLAB program to find L.T of the causal signal x(t)=at

13. Write a MATLAB program to find L.T of the causal signal x(t)=10e^-at

14. Write a MATLAB program to find L.T of the  signal x(t)=1

15. Write a MATLAB program to find L.T of the causal signal x(t)=impulse function

16. Write a MATLAB program to find L.T of the causal signal x(t)=Delta function

17. Write a MATLAB program to find L.T of the causal signal x(t)=cos(at)

18. Write a MATLAB program to find L.T of the causal signal x(t)=sin(at)

19. Write a MATLAB program to find L.T of the causal signal x(t)=e^atcos(at)

20. Write a MATLAB program to find L.T of the causal signal x(t)= e^atsin(at)

21. .Write a MATLAB program to find the auto correlation using L.T.

22. Write a MATLAB program to find L.T of the causal signal x(t)=5t

23. Write a MATLAB program to find L.T of the causal signal x(t)=5e^-at

24. Write a MATLAB program to find L.T of the causal signal x(t)=5e^at

25. Write a MATLAB program to find L.T of the causal signal x(t)=5te^at

Real Time Applications:

Laplace transforms having following applications

	Analysis of differential equations

	Laplace transform spectroscopy

	Quantum mechanics

	Signal processing applications etc

EXPERMENT NO:12

LOCATING THE ZEROS AND POLES AND PLOTTING THE

POLE ZERO MAPS IN S-PLANE AND Z-PLANE FOR THE GIVEN

TRANSFER FUNCTION.

AIM: -

To locating the zeros and poles and plotting the pole zero maps in s-plane and z-plane

for  the given transfer function.

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

A Transfer Function is the ratio of the output of a system to the input of a system,

in the Laplace domain considering its initial conditions to be zero. If we have an input

function of X(s), and an output function Y(s), we define the transfer function H(s) to be:

transfer function is the Laplace transform of a system‟ s impulse response.

Given a continuous-time transfer function in the Laplace domain, H(s) or a discrete-time one

in the Z-domain, H(z), a zero is any value of s or z such that the transfer function is zero, and

a pole is any value of s or z such that the transfer function is infinite.

Zeros:1. The value(s) for z where the numerator of the transfer function equals zero

2. The complex frequencies that make the overall gain of the filter transfer function

zero.

Poles: 1. The value(s) for z where the denominator of the transfer function equals zero

2. The complex frequencies that make the overall gain of the filter transfer function

infinite.

Z-transforms

the Z-transform converts a discrete time-domain signal, which is a sequence of real or

complex numbers, into a complex frequency-domain representation.The Z-transform, like

many other integral transforms, can be defined as either a one-sided or two-sided

transform.

Bilateral Z-transform

The bilateral or two-sided Z-transform of a discrete-time signal x[n] is the function X(z)

defined as

Unilateral Z-transform

Alternatively, in cases where x[n] is defined only for n ≥ 0, the single-sided or unilateral

Z-transform is defined as

In signal processing, this definition is used when the signal is causal.

The roots of the equation P(z) = 0 correspond to the 'zeros' of X(z)

The roots of the equation Q(z) = 0 correspond to the 'poles' of X(z)

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

%locating poles of zero on s-plane%

clc;

clear all;

close all;

num=input('enter numerator co-efficients');

den=input('enter denominator co-efficients');

h=tf(num,den);

poles=roots(den);

zeros=roots(num);

sgrid;

pzmap(h);

grid on;

title('locating poles of zeros on s-plane');

%locating poles &zeros on z-plane%

clc;

clear all;

close all;

num=input('enter numerator coefficient');

den=input('enter denominator coefficient');

p=roots(den);

z=roots(num);

zplane(p,z);

grid;

title('locating poler and zeros on s-plane');

OUTPUT:-

locating poles of zero on s-plane

enter numerator coefficient[1 2 3]

enter denominator coefficient[7 6 5]

locating poles &zeros on z-plane

enter numerator coefficient[1 2 3]

enter denominator coefficient[1 2 3]

RESULT:-

In this experiment the zeros and poles and plotting the pole zero

maps in s-plane and z-plane for the given transfer function

using MATLAB.

VIVA QUESTIONS:-

1.study the details of ztrans() and iztrans() functions?

2.what are poles and zeros?

3.how you specify the stability based on poles and zeros?

4.define s-plane and z-plane?

5.what is the difference b/w s-plane and z-plane?

6.bilateral z-transform

7.z-transforms

8.unilateral z-transforms

9.what is pole?

10.what is zero?

11.what is splane?

12.what is zplane?

13.what is difference between ploe and zero?

14.what is subplot?

15.what is subplot(4,5,6)?

16.define s plane?

17.define z plane?

18.define transform function?

19.what is plot?

20.what is stem?

21.what is difference between plot and stem?

22.what is subplot(2 2 2)?

23.what is system bandwidth?

24what is signal bandwidth

25.what os signal?

26.what is orthogonality function?

27.what are the classifications of sgnals?

22.what are poles and zeros?

29.how you specify the stability based on poles and zeros?

30.define s-plane and z-plane?

31.what is the difference b/w s-plane and z-plane?

32. what is subplot?

33.define transform function?

34..what is plot?

35.what is stem?

36.what is difference between plot and stem?

37.what is fourier transorm?

38.what is system?

39.what is signal?

40.what is trignometric series?

41.application of convolution?

42.applications of lti system?

43.what is stable system?

44.define laplace-transform?

45. what is the condition for convergence of the l.t?

46. what is the region of convergence(roc)?

47. state the shifting property of  l.t?

48. state convolution property of  l.t?

49.define i.l.t?

50.applications of lti system

EXERCISE PROGRAMS

1.Write a MATLAB program to find the impulse response of the following difference equation

3y(n)-5y(n-1)+4y(n-2)=x(n)-2x(n-1).

2. Write a MATLAB program to find the impulse response of the following difference equation

6y(n)-9y(n-1)-4y(n-2)=x(n)-5x(n-1).

3. Write a MATLAB program to find the impulse response of the following difference equation

8y(n)-6y(n-1)+4y(n-2)=x(n)+2x(n-1).

4. Write a MATLAB program to find the impulse response of the following difference equation

3y(n)+5y(n-1)+9y(n-2)=x(n)-9x(n-1).

5. Write a MATLAB program to find the impulse response of the following difference equation

6y(n)-5y(n-1)+4y(n-2)=x(n)-2x(n-1).

6. Write a MATLAB program to find the impulse response of the following difference equation

7y(n)-5y(n-1)+4y(n-2)=x(n)-9x(n-1).

7. Write a MATLAB program to find the impulse response of the following difference equation

3y(n)-5y(n-1)+4y(n-2)=x(n)-6x(n-1).

8. Write a MATLAB program to find the impulse response of the following difference equation

5y(n)-5y(n-1)+5y(n-2)=x(n)-2x(n-1).

9. Write a MATLAB program to find the impulse response of the following difference equation

3y(n)-6 y(n-1)+4y(n-2)=x(n)-7x(n-1).

10. Write a MATLAB program to find the impulse response of the following difference equation

2y(n)-5y(n-1)+4y(n-2)=x(n)-2x(n-1).

11.Write a MATLAB program to find the impulse response of the following difference equation

3y(n)-5y(n-1)+4y(n-2)=5x(n)-6x(n-1).

12. Write a MATLAB program to find the impulse response of the following difference equation

6y(n)-8y(n-1)-4y(n-2)=7x(n)-5x(n-1).

13. Write a MATLAB program to find the impulse response of the following difference equation

8y(n)-8y(n-1)+9y(n-2)=x(n)+5x(n-1).

14. Write a MATLAB program to find the impulse response of the following difference equation

3y(n)+9y(n-1)+9y(n-2)=9x(n)-5x(n-1).

15. Write a MATLAB program to find the impulse response of the following difference equation

6y(n)-5y(n-1)+4y(n-2)=7x(n)-2x(n-1).

16. Write a MATLAB program to find the impulse response of the following difference equation

7y(n)-5y(n-1)+4y(n-2)=x(n)-7x(n-1).

17. Write a MATLAB program to find the impulse response of the following difference equation

3y(n)-5y(n-1)+4y(n-2)=x(n)-6x(n-1).

18. Write a MATLAB program to find the impulse response of the following difference equation

5y(n)-5y(n-1)+5y(n-2)=7x(n)-2x(n-1).

19. Write a MATLAB program to find the impulse response of the following difference equation

3y(n)-6 y(n-1)+4y(n-2)=4x(n)-x(n-1).

20. Write a MATLAB program to find the impulse response of the following difference equation

2y(n)-5y(n-1)+4y(n-2)=x(n)-2x(n-1).

21. Write a MATLAB program to find the impulse response of the following difference equation

7y(n)-5y(n-1)+y(n-2)=x(n)-7x(n-1).

22. Write a MATLAB program to find the impulse response of the following difference equation

3y(n)-5y(n-1)+4y(n-2)=x(n)-6x(n-1)+u(n)

23. Write a MATLAB program to find the impulse response of the following difference equation

5y(n)-5y(n-1)+5y(n-2)=7x(n)-2x(n-1)+u(n).

24. Write a MATLAB program to find the impulse response of the following difference equation

3y(n)-6 y(n-1)+4y(n-2)=4x(n)-x(n-1)+u(n).

25. Write a MATLAB program to find the impulse response of the following difference equation

2y(n)-5y(n-1)+4y(n-2)=x(n)-2x(n-1)+u(n).

Real Time Applications:

Z- Transforms having following applications

	Analysis of differential equations

	Z- transform spectroscopy

	Quantum mechanics

	Signal processing applications etc

	Conversion of analog signals to digital signals etc

EXPERMENT NO:13

GAUSSIAN NOISE

AIM: -

To Generate Gaussian Noise and to Compute its Mean, M.S. Values, Skew, kurtosis,

PS and PDF.

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

Gaussian noise is statistical noise that has a probability density function (abbreviated pdf) of the normal distribution (also known as Gaussian distribution). In other words, the values tha the noise can take on are Gaussian-distributed. It is most commonly used as additive white

noise to yield additive white Gaussian noise (AWGN).Gaussian noise is properly defined as the noise with a Gaussian amplitude distribution.says nothing of the correlation of the noise in time or of the spectral density of the noise.Labeling Gaussian noise as 'white' describes the correlation of the noise. It is necessary to use the term "white Gaussian noise" to be correct. Gaussian noise is sometimes misunderstood tobe white Gaussian noise, but this is not the case.

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

%Gaussion noise%

clc;

close all;

clear all;

N=input('enter no of samples');

R1=rand(1,N);

M=mean(R1);

v=var(R1);

x=psd(R1);

subplot(2,2,1);

plot(R1);

xlabel('sample numbers');

ylabel('amplitude');

title('normal distributed random signal');

subplot(2,2,2);

hist(R1);

xlabel('sample numbers');

ylabel('total');

title('histogram of a random signal');

subplot(2,2,[3,4]);

plot(x);

xlabel('sample numbers');

ylabel('amplitude');

title('psd of a normal random signal');

OUTPUT:-

Gaussion noise

enter no of samples50

RESULT:-

Additive White Gaussian noise was generated and its PSD and PDF were plotted

and its Mean, Standard Deviation, Kurtosis, Skew were Computed using MATLAB

functions.

VIVA QUESTIONS:-

1.What is a noise ?

2.What is gaussion noise?

3.What is correlation ?

4.State Paeseval’s energy theorem for a periodic signal?

5.What is Signum function?

6. How many types of correlation are there?

7. how many types of noises are there?

8.what is auto correlation?

9.what are the types of noise?

10.what is ramp signal?

11.what is signal

12.what is white noise?

13.what is periodic signal?

14.what is non periodic signal?

15.what is system bandwidth?

16.what is signal bandwidth?

17.what is causal system?

18.what is non causal system?

19.what is periodic signal?

20.what is non periodic signal?

21.what is signum function?

22.what is sinc function?

23.what are the types of correlation?

24.what is ramp function?

25.what is correlation ?

26.application of periodic function?

27.what is gaussian noise?

28.what is continuous time signal?

29.what is descrete time signal?

30.what is plot?

31.what is stem?

32.what is time delay?

33.what is xalbel?

34.what is y label?

35.what is psd?

36.what is random signal?

37.what is signal?

38.what are the types of signals?

39.what is bandwidth?

40.what is correlation?

41.what are the  correlation types?

42.what is y label?

43.what is mean square?

44.what is periodi?

45.what is non periodic signal?

46..what is gaussian noise?

47.what is continuous time signal?

48.what is descrete time signal?

49.what is plot?

50.what is time delay?

EXERCISE PROGRAMS

1.Write a MATLAB program to plot the co-sinusoidal signal.

2. Write a MATLAB program to subtract two sinusoidal signals.

3. Write a MATLAB program to subtract and multiply twosinusoidal signals.

4. Write a MATLAB program to right shift the signal to 5 times of the original signal.

5. Write a MATLAB program to left shift the signal to 8 times of the original signal.

6. Write a MATLAB program to add two different signals with 2 <t<5

7. Write a MATLAB program to shift a positive time line signal to negative timeline signal.

8. Write a MATLAB program to subtract co-sinusoidal signals.

9. Write a MATLAB program to subtract two sinusoidal signals

10. Write a MATLAB program to division and multiply two co-sinusoidal signals.

11. Write a MATLAB program to generate time scaling of a sequence.

12. Write a MATLAB program to generate time shifting of a sequence.

13. Write a MATLAB program to generate time folding of a sequence.

14. Write a MATLAB program to generate amplitude scaling of a sequence with amplitude 5.

15. Write a MATLAB program to generate time scaling of a sequence with time 2sec.

16. Write a MATLAB program to add two different signals with 4 <t<8

17. Write a MATLAB program to shift a negative time line signal to positive timeline signal.

18. Write a MATLAB program to subtract sinusoidal signals.

19. Write a MATLAB program to subtract and divide two sinusoidal signals

20. Write a MATLAB program to add and multiply two co-sinusoidal signals.

21. Write a MATLAB program to generate time scaling of a signal.

22. Write a MATLAB program to generate time shifting of a signal.

23. Write a MATLAB program to generate time folding of a signal.

24. Write a MATLAB program to generate amplitude scaling of a sequence with amplitude 10.

25. Write a MATLAB program to generate time scaling of a sequence with time 5sec.

Real Time Applications:

•	Stream processing

•	Block processing

•	Vector processing

•	Digital  processing applications

EXPERMENT NO:14

SAMPLING THEORM

AIM: -

To Demonstrate Sampling Theorem and aliasing effect using MATLAB.

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

Sampling Theorem:

The theorem shows that a band limited analog signal that has been sampled can be perfectly

reconstructed from an infinite sequence of samples if the sampling rate exceeds 2B samples

per second, where B is the highest frequency in the original signal.

If a signal contains a component at exactly B hertz, then samples spaced at exactly 1/(2B)

seconds do not completely determine the signal, Shannon's statement notwithstanding.

Proof: Let g(t) be a bandlimited signal whose bandwidth is fm

(wm = 2πfm).

Figure 2: (a) Original signal g(t) (b) Spectrum G(w)

δ (t) is the sampling signal with fs = 1/T > 2fm.

Figure 3: (a) sampling signal δ (t) ) (b) Spectrum δ (w)

Let gs(t) be the sampled signal. Its Fourier Transform Gs(w) isgiven by

Figure 4: (a) sampled signal gs(t) (b) Spectrum Gs(w)

To recover the original signal G(w):

1. Filter with a Gate function, H2wm(w) of width 2wm

Scale it by T.

Figure 5: Recovery of signal by filtering with a fiter of width 2wm

Aliasing

{ Aliasing is a phenomenon where the high frequency components of the sampled signal

interfere with each other because of inadequate sampling ws < 2wm.

Figure 6: Aliasing due to inadequate sampling

Aliasing leads to distortion in recovered signal. This is the

reason why sampling frequency should be atleast twice thebandwidth of the signal.

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

%sampling theorem%

clc;

clear all;

close all;

t=0:0.001:1;

f=5;

x=cos(2*pi*f*t);

plot(t,x);

%when fs>2fm%

fs1=3*f;

ts1=1/fs1;

n1=0:ts1:1;

xs1=cos(2*pi*f*n1);

figure;

plot(n1,xs1,'r');

hold on;

plot(t,x);

hold off;

%when fs=2fm%

fs2=2*f;

ts2=1/fs2;

n2=0:ts2:1;

xs2=cos(2*pi*f*n2);

figure;

plot(n2,xs2,'r');

hold on;

plot(t,x);

hold off;

%when fs<2fm%

fs3=1.5*f;

ts3=1/fs3;

n3=0:ts3:1;

xs3=cos(2*pi*f*n3);

figure;

plot(n3,xs3,'r');

hold on;

plot(t,x);

hold off;

OUTPUT:-

sampling theorem

when fs>2fm

when fs=2fm

when fs<2fm

RESULT:-

In this experiment the sampling theorem have been verified

using MATLAB.

VIVA QUESTIONS:-

sampling Theorem

- If Sampled frequency less than nyquist rate then it is called as.

- If Sampled frequency greater  than nyquist rate then it is called as.

- If Sampled frequency is nyquist rate then it is called as.

- What is the cause of Aliasing effect.

- Explain the function of Anti aliasing filter.

- What are types of sampling techniques are there.

- The spectrum of the sampled signal may be obtained without overlapping only if

- The desired signal of maximum frequency wm centered at frequency w=0 may be recovered if

- A distorted signal of frequency fm is recovered from a sampled signal if the sampling frequency fs is

- Calculate the minimum sampling rate to avoid aliasing when a continuous time signal is given by x(t) = 5 cos 400πt

- Calculate the minimum sampling rate to avoid aliasing when a continuous time signal is given by x(t) = 5 sin400πt

- Calculate the minimum sampling rate to avoid aliasing when a continuous time signal is given by x(t) = 5 cos 400πt +5sin 400πt

- Calculate the Nyquist rate for sampling when a continuous time signal is given by
x(t) = 5 cos 100πt +10 cos 200πt – 15 cos 300πt.

- What is low pass filter.

- Define  high pass filter.

- Explain  Band  pass filter.

- What is Band stop  pass filter.

- What is band reject filter.

- The instantaneous sampling means?

- The sampling technique that has the minimum noise interference is.

- Explain clc command.

- Narrate the term clear in MATLAB.

- Draw the Frequency response of high  pass filter.

- Draw the Frequency response of Band  pass filter.

- Draw the Frequency response of Band  pass filter.

- Define aliasing effect.

- suggest the method to eliminate aliasing effect.

- compare sampling techniques.

- Distinguish between sampling rate and sampling frequency.

- Define sampling Interval

- Explain cross correlation.

- how to do the sampling process.

- write the differences between continuous and discrete signals.

- Write the equation of sampled signal.

- write the formula for natural sampling.

- calculate Nyquist rate for Band limited signals.

- Define cut off frequency.

- Explain roll off rate.

- Define the term SNR.

- Draw the Flat to sampled signal.

- Distingush between Natural and Ideal sampling.

- what is meant by Ideal sampling.

- Narrate the term Instantaneous sampling.

- Explain the commands plot and subplot.

- Write The properties of psd.

- Write the applications of sampling.

- What are the advantages of sampling

- Explain Draw backs in Analog signal processing.

- mention the method to avoid Interference.

- Elaborate the term white noise.

EXERCISE PROGRAM:-

1.	Write a MATLAB program to find the effect of up sampling in frequency domain.

2.	 Write a matlab program to get the above output where t0 = 2

3.	 Write a MATLAB program to find the effect of up sampling in frequency domain for 20khz.

4.	 Write a program to get the following output t = -1: -5

5.

Write  mat lab program to get the above output

X(t) = 1; (0<t<2)

= 10;(2<t<15)   Note: Add ramp and unit

6.	Write a matlab program to generate a sine wave with amplitude = 3, frequency 20Hz.

7.	Write a matlab program to generate a cos wave with amplitude = 3, frequency 20Hz.

8.	Write a matlab program to generate a triangular  wave with amplitude = 8, frequency 10Hz.

9.	Write a matlab program to generate a square  wave with amplitude = 2, frequency 10kHz.

10.	Write a matlab program to generate the signum function.

11.	Write a matlab program to generate exponential growing signal.

12.	Write a matlab program to generate exponential decaying signal.

13.	Write a matlab program to generate a triangular wave with amplitude = 6,frequency 1Hz.

14.	Write a matlab program to generate a square  wave with amplitude = 5, frequency 5kHz.

15.	Write a matlab program to generate a sawtooth wave with amplitude = 8, frequency5Khz.

16.	Write a matlab program to generate a sine wave with amplitude = 5, frequency 5Hz.

17.	Write a matlab program to generate a cos wave with amplitude = 4, frequency 10Hz.

18.	Write a matlab program to generate a triangular wave with amplitude = 5, frequency 4Hz.

19.	Write a matlab program to generate a square  wave with amplitude = 1, frequency 20kHz.

20.	Write a matlab program to generate the signum function.

21.	Wr ite a matlab program to generate a square wave with amplitude = 4, frequency 5kHz.

22.	Write a matlab program to generate a triangular  wave with amplitude = 6, frequency 6Hz.

23.	Write a matlab program to generate a triangular wave with amplitude = 8,frequency 2Hz.

24.	Write a matlab program to generate a square  wave with amplitude = 7, frequency 5kHz.

25.	Write a matlab program to generate a sawtooth wave with amplitude =10, frequency 10Khz.

Real Time Applications:

- Industrial control and automation (Control the velocity or position of an object)

- Examples: Controlling the position of a valve or shaft of a motor

- Examples: Digital processing applications

- TV reception

- RADAR

- SONAR

EXPERMENT NO:15

REMOVAL OF NOISE BY AUTO CORRELATION/CROSS

CORRELATION

AIM: -

removal of noise by auto correlation/cross correlation.

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

Detection of a periodic signal masked by random noise is of greate importance .The

noise signal encountered in practice is a signal with random amplitude variations. A

signal is uncorrelated with any periodic signal. If s(t) is a periodic signal and n(t) is a

noise signal then

T/2	1/T

Lim ∫ S(t)n(t-T) dt=0  for all T

T∞ -T/2

Qsn(T)= cross correlation function of s(t) and n(t) Then Qsn(T)=0

Detection by Auto-Correlation:

S(t)=Periodic Signal mixed with a noise signal n(t).Then f(t) is [s(t ) + n(t) ]

Let Qff(T) =Auto Correlation Function of f(t)

Qss(t) = Auto Correlation Function of S(t)

Qnn(T) = Auto Correlation Function of n(t)

T/2

Qff(T)= Lim 1/T ∫ f(t)f(t-T) dt

T->∞      -T/2

T/2

= Lim 1/T ∫ [s(t)+n(t)][s(t-T)+n(t-T)] dt

T--∞    -T/2

=Qss(T)+Qnn(T)+Qsn(T)+Qns(T)

The periodic signal s(t) and noise signal n(t) are uncorrelated

Qsn(t)=Qns(t)=0 ;Then Qff(t)=Qss(t)+Qnn(t)

The Auto correlation function of a periodic signal is periodic of the same frequency and

the Auto correlation function of a non periodic signal is tends to zero for large value of T

since s(t) is a periodic signal and n(t) is non periodic signal so Qss(T) is a periodic where

as aQnn(T) becomes small for large values of T Therefore for sufficiently large values of

T Qff(T) is equal to Qss(T).

Detection by Cross Correlation:

f(t)=s(t)+n(t)

c(t)=Locally generated signal with same frequencyas that of S(t)

T/2

Qfc (t) = Lim 1/T ∫ [s(t)+n(t)] [ c(t-T)] dt

T--∞ -T/2

= Qsc(T)+Qnc(T)

C(t) is periodic function and uncorrelated with the random noise signal n(t). Hence

Qnc(T0=0) Therefore Qfc(T)=Qsc(T)

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

%auto correlation%

clc;

clear all;

close all;

t=0:0.1:pi*4;

s=sin(t);

k=2;

subplot(6,1,1)

plot(s);

title('signal s');

xlabel('t');

ylabel('amplitude');

n = randn([1 126]);

f=s+n;

subplot(6,1,2)

plot(f);

title('signal f=s+n');

xlabel('t');

ylabel('amplitude');

as=xcorr(s,s);

subplot(6,1,3)

plot(as);

title('auto correlation of s');

xlabel('t');

ylabel('amplitude');

an=xcorr(n,n);

subplot(6,1,4)

plot(an);

title('auto correlation of n');

xlabel('t');

ylabel('amplitude');

cff=xcorr(f,f);

subplot(6,1,5)

plot(cff);

title('auto correlation of f');

xlabel('t');

ylabel('amplitude');

hh=as+an;

subplot(6,1,6)

plot(hh);

title('addition of as+an');

xlabel('t');

ylabel('amplitude');

%CROSS CORRELATION %

clc;

clear all;

close all;

t=0:0.1:pi*4;

s=sin(t);

k=2;

%sk=sin(t+k);

subplot(7,1,1)

plot(s);

title('signal s');

xlabel('t');ylabel('amplitude');

c=cos(t);

subplot(7,1,2)

plot(c);

title('signal c');

xlabel('t');

ylabel('amplitude');

n = randn([1 126]);

f=s+n;

subplot(7,1,3)

plot(f);

title('signal f=s+n');

xlabel('t');

ylabel('amplitude');

asc=xcorr(s,c);

subplot(7,1,4)

plot(asc);

title('auto correlation of s and c');

xlabel('t');ylabel('amplitude');

anc=xcorr(n,c);

subplot(7,1,5)

plot(anc);

title('auto correlation of n and c');

xlabel('t');ylabel('amplitude');

cfc=xcorr(f,c);

subplot(7,1,6)

plot(cfc);

title('auto correlation of f and c');

xlabel('t');ylabel('amplitude');

hh=asc+anc;

subplot(7,1,7)

plot(hh);

title('addition of asc+anc');

xlabel('t');ylabel('amplitude');

OUTPUT:-

auto correlation

CROSS CORRELATION

RESULT:-

In this experiment the removal of noise by autocorrelation/cross correlation have

been verified using MATLAB.

VIVA QUESTIONS:-

- What is the The value of probability density function of random variable.

- Random process is a function of what variable.

- When a  random process is called as stationary .

- Autocorrelation function is maximum at what value.

- How to denote Standard deviation.

- What is the The average power of white noise.

- Measure the  mean and variance of white noise .

- What does the set comprising all possible outcomes of an experiment known as ?

- What does an each outcome in the sample space regarded as ?

- Explain the term Mutually exclusive events.

- What would be the probability of an event 'G' if G denotes its complement, according to the axioms of probability?

- What would happen if the two events are statistically independent ?

- What would be the joint probability of statistically independent events that occur simultaneously ?

- What is the value of an area under the conditional PDF ?

- When do the conditional density functions get converted into the marginally density functions ?

- Which among the below mentioned standard PDFs is/are applicable to discrete random variables ?

- What would happen if the value of term [(m-x) / (σ √2)] increases in the expression of Guassian CDF?

- Which type of standard PDFs has/ have an ability to describe an integer valued random variable concerning to the repeated trials carried /conducted in an experiment?

- Which theorem states that the total average power of a periodic signal is equal to the sum of average powers of the individual fourier coefficients?

- What does the spectral density function of any signal specify?

- Which among the below mentioned transform pairs is/are formed between the auto-correlation function and the energy spectral density, in accordance to the property of Energy Spectral Density (ESD)?

- Which function has a provision of determining the similarity between the signal and its delayed version?

- Narrate the term clear in MATLAB.

- Draw the Frequency response of high  pass filter.

- What is meant by MATLAB.

- How to define 2x2 matrix in MATLAB.

- How to find Inverse of the matrix.

- Mention the procedure to find corrrelation.

- Distingush between Autocorrelation and cross correlation.

- Explain the command Xcorr.

- Differentiate between plot and subplot.

- Write the equation of sampled signal.

- What is meant by continuous Random sequence.

- calculate Nyquist rate for Band limited signals.

- calculate the value of  autocorrelation value at origin.

- Write about properties of correlation.

- Describe the  properties of  Auto correlation.

- Write about properties cross correlation.

- What is meant by continuous Random process.

- Explain  Discrete Random process.

- What is meant by Discrete Random sequence

- write the formula for natural sampling.

- What is low pass filter.

- Define  high pass filter.

- Explain  Band  pass filter.

- Write the properties of LTI system.

- compare causal and Non causal systems.

- Explain Delay Line Canceller.

- Narrate the term pole.

- What is meant by zero.

EXERCISE PROGRAMS

1.Write a MATLAB program to find the cross correlation using FFT.

2. Write a mat lab program to perform the cross correlation between sinusoidal and ramp function and see how mat lab reacts to it.

3. Write a MATLAB program to perform cross correlation between square and step signal and see how mat lab reacts to it.

4. Write a MATLAB program to perform cross correlation between sinusoidal and ramp signal and see how mat lab reacts to it.

5. Write a MATLAB program to perform the cross correlation between X (n) = [1 2 3 5] and y (n) = [- 1 – 2] and see how matlab reacts to it.

6. Write a MATLAB program to perform the cross correlation between X (n) = [1 -3  5] and y (n) = [1 2 3 4] and see how matlab reacts to it.

7. Write a MATLAB program to perform the cross correlation between X (n) = [1 0 1 1] and y (n) = [1 0 0 0 0] and see how matlab reacts to it.

8.Write a MATLAB program to perform the cross correlation between X (n) = [1 1 1 1 0 0 0 0] and y (n) = [1 0 1 0 1 0 1 0] and see how matlab reacts to it.

9.Write a MATLAB program to perform the cross correlation between X (n) = [1 1 0 0] and y (n) = [1 1 1 1] and see how matlab reacts to it.

10.Write a MATLAB program to perform the cross correlation between X (n) = [6 7 8 9 10] and y (n) = [5 4 3 2 1] and see how matlab reacts to it.

11.Write the MATLAB program to perform cross correlation between the following sequences

X(n)=[2 -2 3 2, h(n) = [-2 4 -5 2].

12. Write a mat lab program to perform the convolution between step and ramp function and see how mat lab reacts to it.

13. Write a MATLAB program to perform convolution between square and step signal and see how mat lab reacts to it.

14. Write a MATLAB program to perform convolution between sinusoidal and square signal and see how mat lab reacts to it.

15. Write a MATLAB program to perform the convolution between X (n) = [2 3 4 5] and y (n) = [-2  –4] and see how matlab reacts to it.

16. Write a MATLAB program to perform the convolution between X (n) = [3 -4 6] and y (n) = [2 6 8 6] and see how matlab reacts to it.

17. Write a MATLAB program to perform the convolution between X (n) = [3 1 2 0] and y (n) = [2 4 3 4 5] and see how matlab reacts to it.

18.Write a MATLAB program to perform the convolution between X (n) = [2 2 1 0 2 1 1 0] and y (n) = [1 1 1 2 1 0 1 0] and see how matlab reacts to it.

19.Write a MATLAB program to perform the convolution between X (n) = [2 1 2 1] and y (n) = [1 2 1 2] and see how matlab reacts to it.

20.Write a MATLAB program to perform the convolution between X (n) = [5 6 9 8 7] and y (n) = [1 2 3 2 1] and see how matlab reacts to it.

21.Write the MATLAB program to perform cross correlation between the following sequences

X(n)=[4 -6 5 1], h(n) = [-1 3 -4 3].

22. Write a mat lab program to perform the cross correlation between ramp and step function and see how mat lab reacts to it.

23. Write a MATLAB program to perform cross correlation between square and step signal and see how mat lab reacts to it.

24. Write a MATLAB program to perform cross correlation between sinusoidal and step signal and see how mat lab reacts to it.

25. Write a MATLAB program to perform the convolution between X (n) = [1 5 6 5] and y (n) = [-3 –6] and see how matlab reacts to it.

Real Time Applications:

•	The cross correlation is to determine the response y[n] of a system of a known impulse response h[n] for a given input signal x[n] to obtain y[n].

Digital signal processing applications etc…

EXPERMENT NO:16

EXTRACTION OF PERIODIC SIGNAL MASKED BY NOISE USING CORRELATION

Extraction of Periodic Signal Masked By Noise Using Correlation

AIM: -

To generate a periodic sequence, corrupt it with zero mean White noise and extract the

sequence using periodic circular cross correlation.

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

Removal of noise by Filtering is not the solution because noise contains power at all

frequencies. Hence we use correlation techniques.

The theorem is useful for analyzing linear time-invariant systems, LTI systems, when the inputs and outputs are not square integrable, so their Fourier transforms do not exist. A corollary is that the Fourier transform of the autocorrelation function of the output of an LTI system is equal to the product of the Fourier transform of the autocorrelation function of the input of the system times the squared magnitude of the Fourier transform of the system impulse response. This works even when the Fourier transforms of the input and output signals do not exist because these signals are not square integrable, so the system inputs and outputs cannot be directly related by the Fourier transform of the impulse response. Since the Fourier transform of the autocorrelation function of a signal is the power spectrum of the signal, this corollary is equivalent to saying that the power spectrum of the output is equal to the power spectrum of the input times the power transfer function. This corollary is used in the parametric method for power spectrum estimation.

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

clc;

clear all;

close all;

n=256;

k1=0:n-1;

x=cos(32*pi*k1/n)+sin(48*pi*k1/n);

plot(k1,x);

%Module to find period of input signal

k=2;

xm=zeros(k,1);

ym=zeros(k,1);

hold on;

for i=1:k

[xm(i) ym(i)]=ginput(1);

plot(xm(i), ym(i),'r*');

end;

period=abs(xm(2)-xm(1));

rounded_p=round(period);

m=rounded p;

% Adding noise and plotting noisy signal

y=x+randn(1,n);

figure;

plot(k1,y);

% To generate impulse train with the period as that of input signal%

d=zeros(1,n);

for i=1:n

if (rem(i-1,m)==0)

d(i)=1;

end;

end;

% correlating noisy signal and impulse train

cir=cxcorr1(y,d);

%plotting the original and reconstructed signal

m1=0:n/4;

figure

plot(m1,x(m1+1),'r',m1,m*cir(m1+1));

OUTPUT:-

RESULT:-

Using Cross- correlation the periodic signal from noise was estimated using

MATLAB.

VIVA QUESTIONS:-

- Which type of standard PDFs has/ have an ability to describe an integer valued random variable concerning to the repeated trials carried /conducted in an experiment?

- Which theorem states that the total average power of a periodic signal is equal to the sum of average powers of the individual fourier coefficients?

- What does the spectral density function of any signal specify?

- Which among the below mentioned transform pairs is/are formed between the auto-correlation function and the energy spectral density, in accordance to the property of Energy Spectral Density (ESD)?

- Which function has a provision of determining the similarity between the signal and its delayed version?

- Which property is exhibited by the auto-correlation function of a complex valued signal?

- What is the value of an area under the conditional PDF ?

- When do the conditional density functions get converted into the marginally density functions ?

- What would happen if the value of term [(m-x) / (σ √2)] increases in the expression of Guassian CDF?

- Which property of delta function indicates the equality between the area under the product of function with shifted impulse and the value of function located at unit impulse instant ?

- Elaborate Damped sinusoids.

- A system is said to be shift invariant only if?

- Which condition determines the causality of the LTI system in terms of its impulse response ?

- Under which conditions does an initially relaxed system become unstable ?

- Which block of the discrete time systems requires memory in order to store the previous input?

- Explain the term periodic signal.

- What is the condition for a signal  to be periodic.

- Find the period of the signal x(t)=cos2t

- Find the period of the signal x(t)=sin4t

- Find the period of the signal x(t)=ej2t

- What is meant by Non periodic signal.

- Distinguish between periodic and A periodic signals.

- In general periodic signals are called as?

- In general  aperiodic signals are called as?

- What is the value of power of periodic signal.

- write the value of power of Energy signal.

- Calculate the energy of power signal.

- Find the power of energy signal.

- Define sampling Interval

- Explain cross correlation.

- how to do the sampling process.

- write the differences between continuous and discrete signals.

- Write the equation of sampled signal.

- Explain clc command.

- Narrate the term clear in MATLAB.

- Draw the Frequency response of high  pass filter.

- What is Band stop  pass filter.

- What is band reject filter.

- The instantaneous sampling means?

- The sampling technique that hasthe minimum noise interference is.

- What Types of analog pulse modulation systems.

- Write about correlation.

- Write about Auto correlation.

- Explain Draw backs in Analog signal processing.

- mention the method to avoid Interference.

- Elaborate the term white noise.

- Is Autocorrelation function is an even or odd function.

- Justify How Autocorrelation function is Even function.

- How to measure even  part of the signal.

- Write the applications of Random process.

EXERCISE PROGRAM:-

1.Simulate a matlab program on autocorrelation and PSD if x = [1 2 3 4 5 ].

2. Write a mat lab program to perform the cross correlation between sinusoidal and ramp function and see how mat lab reacts to it.

3. Write a MATLAB program to perform cross correlation between square and step signal and see how mat lab reacts to it.

4. Write a MATLAB program to perform cross correlation between sinusoidal and ramp signal and see how mat lab reacts to it.

5. Write a MATLAB program to perform the cross correlation between X (n) = [1 2 3 5] and y (n) = [- 1 – 2] and see how matlab reacts to it.

6. Write a MATLAB program to perform the cross correlation between X (n) = [1 -3  5] and y (n) = [1 2 3 4] and see how matlab reacts to it.

7. Write a MATLAB program to perform the cross correlation between X (n) = [1 0 1 1] and y (n) = [1 0 0 0 0] and see how matlab reacts to it.

8.Write a MATLAB program to perform the cross correlation between X (n) = [1 1 1 1 0 0 0 0] and y (n) = [1 0 1 0 1 0 1 0] and see how matlab reacts to it.

9.Write a MATLAB program to perform the cross correlation between X (n) = [1 1 0 0] and y (n) = [1 1 1 1] and see how matlab reacts to it.

10.Write a MATLAB program to perform the cross correlation between X (n) = [6 7 8 9 10] and y (n) = [5 4 3 2 1] and see how matlab reacts to it.

11.Write the MATLAB program to perform cross correlation between the following sequences

X(n)=[2 -2 3 2, h(n) = [-2 4 -5 2].

12. Write a mat lab program to perform the convolution between step and ramp function and see how mat lab reacts to it.

13. Write a MATLAB program to perform convolution between square and step signal and see how mat lab reacts to it.

14. Write a MATLAB program to perform convolution between sinusoidal and square signal and see how mat lab reacts to it.

15. Write a MATLAB program to perform the convolution between X (n) = [2 3 4 5] and y (n) = [-2  –4] and see how matlab reacts to it.

16. Write a MATLAB program to perform the convolution between X (n) = [3 -4 6] and y (n) = [2 6 8 6] and see how matlab reacts to it.

17. Write a MATLAB program to perform the convolution between X (n) = [3 1 2 0] and y (n) = [2 4 3 4 5] and see how matlab reacts to it.

18.Write a MATLAB program to perform the convolution between X (n) = [2 2 1 0 2 1 1 0] and y (n) = [1 1 1 2 1 0 1 0] and see how matlab reacts to it.

19.Write a MATLAB program to perform the convolution between X (n) = [2 1 2 1] and y (n) = [1 2 1 2] and see how matlab reacts to it.

20.Write a MATLAB program to perform the convolution between X (n) = [5 6 9 8 7] and y (n) = [1 2 3 2 1] and see how matlab reacts to it.

21.Write the MATLAB program to perform cross correlation between the following sequences

X(n)=[4 -6 5 1], h(n) = [-1 3 -4 3].

22. Write a mat lab program to perform the cross correlation between ramp and step function and see how mat lab reacts to it.

23. Write a MATLAB program to perform cross correlation between square and step signal and see how mat lab reacts to it.

24. Write a MATLAB program to perform cross correlation between sinusoidal and step signal and see how mat lab reacts to it.

25. Write a MATLAB program to perform the convolution between X (n) = [1 5 6 5] and y (n) = [-3 –6] and see how matlab reacts to it.

Real Time Applications:

- The cross correlation is to determine the response y[n] of a system of a known impulse response h[n] for a given input signal x[n] to obtain y[n].

- Digital signal processing applications etc…

- Tv reception.

EXPERMENT NO:17

VERIFICATION OF WIENER–KHINCHIN RELATION

AIM: -

Verification of wiener–khinchine relation.

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

The Wiener–Khinchin theorem (also known as the Wiener–Khintchine theorem and

sometimes as the Wiener–Khinchin–Einstein theorem or the Khinchin–Kolmogorov

theorem) states that the power spectral density of a wide-sense-stationary random

process is the Fourier transform of the corresponding autocorrelation function.

Continuous case:

Where

is the autocorrelation function defined in terms of statistical expectation, and where is the

power spectral density of the function . Note that the autocorrelation function is defined

in terms of the expected value of a product, and that the Fourier transform of does not

exist in general, because stationary random functions are not square integrable.

The asterisk denotes complex conjugate, and can be omitted if the random process is realvalued.

Discrete case:

where

and where is the power spectral density of the function with discrete values . Being a

sampled and discrete-time sequence, the spectral density is periodic in the frequency

domain.

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

% wiener–khinchine relation%

Clc;

clear all;

t=0:0.1:2*pi;

x=sin(2*t);

subplot(3,2,1);

plot(x);

au=xcorr(x,x);

subplot(3,2,2);

plot(au);

v=fft(au);

subplot(3,2,3);

plot(abs(v));

fw=fft(x);

subplot(3,2,4);

plot(fw);

fw2=(abs(fw)).^2;

subplot(3,2,5);

plot(fw2);

OUTPUT:-

wiener–khinchine relation

RESULT:-

Calculation of Auto-correlation function using Weiner-Khinchin theorm was verified

using MATLAB.

VIVA QUESTIONS:-

Verification of winer-khinchine relations.

- Find the fourier transform of any hyperbolic function

- Power spectral density function is  even function or odd function.

- Real part of Power spectral density function is  even function or odd function.

- Imaginary part of Power spectral density function is  even function or odd function.

- Cross Power spectral density function is  even function or odd function.

- In cross PSD Real part of Power spectral density function is  even function or odd function.

- In cross PSD Imaginary part of Power spectral density function is  even function or odd function.

- Explain the commands plot and subplot.

- Write The properties of psd.

- Write The properties of cross psd.

- Find the fourier transform of sin function

- Find the fourier transform of e-at  u(t)function

- Find the fourier transform of  e-at  u(-t)function

- Find the fourier transform of any hyperbolic function

- Distinguish between sampling rate and sampling frequency.

- Define sampling Interval

- Explain cross correlation.

- how to do the sampling process.

- write the differences between continuous and discrete signals.

- What is Band stop  pass filter.

- Define  band reject filter.

- What would happen if the two events are statistically independent ?

- What would be the joint probability of statistically independent events that occur simultaneously ?

- What is the value of an area under the conditional PDF ?

- When do the conditional density functions get converted into the marginally density functions ?

- What would happen if the two events are statistically independent ?

- What would be the joint probability of statistically independent events that occur simultaneously ?

- What is the value of an area under the conditional PDF ?

- When do the conditional density functions get converted into the marginally density functions ?

- Mention the procedure to find corrrelation.

- Distingush between Autocorrelation and cross correlation.

- Explain the command Xcorr.

- Differentiate between plot and subplot.

- Write the equation of sampled signal.

- What is meant by continuous Random sequence.

- What is meant by Wiener -Kintchine Relation.

- Justify the Wiener -Kintchine Relation.

- measure the Fourier Transform of Impulse function.

- Distingush between ESD and PSD.

- Elaborate the Term ESD

- Elaborate the Term PSD

- Psd value is maximum at?

- Esd value is maximum at?

- Differentiate energy and ESD.

- compare psd and power.

- Narrate the term spectral density.

- mention advantages Spectral density.

- What is the function of spectrum analyzer.

- write the applications of psd.

- Formulate the relation between ACF and psd

EXERCISE PROGRAMS

1.Write a MATLAB program to illustrate the effect of anti-aliasing filter.

2. Write a MATLAB program to subtract two sinusoidal signals.

3. Write a MATLAB program to subtract and multiply twosinusoidal signals.

4. Write a MATLAB program to right shift the signal to 5 times of the original signal.

5. Write a MATLAB program to left shift the signal to 8 times of the original signal.

6. Write a MATLAB program to add two different signals with 2 <t<5

7. Write a MATLAB program to shift a positive time line signal to negative timeline signal.

8. Write a MATLAB program to subtract co-sinusoidal signals.

9. Write a MATLAB program to subtract two sinusoidal signals

10. Write a MATLAB program to division and multiply two co-sinusoidal signals.

11. Write a MATLAB program to generate time scaling of a sequence.

12. Write a MATLAB program to generate time shifting of a sequence.

13. Write a MATLAB program to generate time folding of a sequence.

14. Write a MATLAB program to generate amplitude scaling of a sequence with amplitude 5.

15. Write a MATLAB program to generate time scaling of a sequence with time 2sec.

16. Write a MATLAB program to add two different signals with 4 <t<8

17. Write a MATLAB program to shift a negative time line signal to positive timeline signal.

18. Write a MATLAB program to subtract sinusoidal signals.

19. Write a MATLAB program to subtract and divide two sinusoidal signals

20. Write a MATLAB program to add and multiply two co-sinusoidal signals.

21. Write a MATLAB program to generate time scaling of a signal.

22. Write a MATLAB program to generate time shifting of a signal.

23. Write a MATLAB program to generate time folding of a signal.

24. Write a MATLAB program to generate amplitude scaling of a sequence with amplitude 10.

25. Write a MATLAB program to generate time scaling of a sequence with time 5sec.

Real Time Applications:

•	Stream processing applications

•	Block processing applications

•	Vector processing applications

•	Signal  processing applications

EXPERMENT NO:18

CHECKING A RANDOM PROCESS FOR STATIONARITY IN WIDE SENSE.

AIM: -

Checking a random process for stationarity in wide sense.

SOFTWARE REQURIED:-

1.MATLAB R2010a.

2.Windows XP SP2.

THEORY:-

A stationary process (or strict(ly) stationary process or strong(ly) stationary process) is a

stochastic process whose joint probability distribution does not change when shifted in time or space. As a result, parameters such as the mean and variance, if they exist, also do not change over time or position..Weak or wide-sense stationary A weaker form of stationary commonly employed in signal processing is known as weak-sense stationary, wide-sense stationarity (WSS) or covariance stationarity. WSS random processes only require that 1st and 2nd moments do not vary with respect to time.

and autocorrelation function

The first property implies that the mean function m x(t) must be constant. The second property implies that the correlation function depends only on the difference between t1 and t2 and only needs to be indexed by one variable rather than two variables. Thus, instead of writing,

Rx(t1-t2,0)we usually abbreviate the notation and write

This also implies that the auto covariance depends only on _ = t1 − t2, since

Stationary and Non Stationary Random Process:

A random X(t) is stationary if its statistical properties are unchanged by a time shift in the time origin. When the auto-Correlation function Rx(t, t+T ) of the random X(t) varies with time difference T and the mean value of the random variable X(t1) is independent of the choicet1,then X(t) is said to be stationary in the wide sense or wide-sense stationary . So a continuous- Time random process X(t) which is WSS has the following

properties

1) E[X(t)]=μX(t)= μX(t+T)

2) The Autocorrelation function is written as a function of T that is

RX(t, t+T)=Rx(T)

If the statistical properties like mean value or moments depends on time then the random process is said to be non-stationary.

When dealing with two random process X(t) and Y(t), we say that they are jointly wide-sense stationary if each process is stationary in the wide-sense.

Rxy(t,t+T)=E[X(t)Y(t+T)]=Rxy(T).

PROCEDURE:-

- Open MATLAB

- Open new M-file

- Type the program

- Save in current directory

- Compile and Run the program

- For the output see command window\ Figure window

PROGRAM:-

Clc;

Close all;

clear all

y = randn([1 40])

my=round(mean(y));

z=randn([1 40])

mz=round(mean(z));

vy=round(var(y));

vz=round(var(z));

t = sym('t','real');

h0=3;

x=y.*sin(h0*t)+z.*cos(h0*t);

mx=round(mean(x));

k=2;

xk=y.*sin(h0*(t+k))+z.*cos(h0*(t+k));

x1=sin(h0*t)*sin(h0*(t+k));

x2=cos(h0*t)*cos(h0*(t+k));

c=vy*x1+vz*x1;

OUTPUT:-

%if we solve "c=2*sin(3*t)*sin(3*t+6)" we get c=2cos(6)

%which is a costant does not depent on variable 't'

% so it is wide sence stationary.

RESULT:-

Checking a random process for stationarity in wide sense by using MATLAB .

VIVA QUESTIONS:-

- What is the value of an area under the conditional PDF ?

- When do the conditional density functions get converted into the marginally density functions ?

- Which among the below mentioned standard PDFs is/are applicable to discrete random variables ?

- What would happen if the value of term [(m-x) / (σ √2)] increases in the expression of Guassian CDF?

- Which type of standard PDFs has/ have an ability to describe an integer valued random variable concerning to the repeated trials carried /conducted in an experiment?

- Which theorem states that the total average power of a periodic signal is equal to the sum of average powers of the individual fourier coefficients?

- How to Represent any function using Fourier series.

- calculate the value of  autocorrelation value at origin.

- Write about properties of correlation.

- Describe the  properties of  Auto correlation.

- Write about properties cross correlation.

- What is meant by continuous Random process.

- Explain  Discrete Random process.

- Narrate the   First order random process.

- Elaborate  second order random process.

- What is meant by third order random process.

- Explain  nth order random process.

- What is meant by  random process.

- Classify the random process

- .What is meant by continuous Random process.

- Write Briefly about  Discrete Random process.

- What is meant by Discrete Random sequence

- What is meant by continuous Random sequence.

- What is meant Ergodicity.

- Explain  Time average.

- Explain cross correlation.

- how to do the sampling process.

- write the differences between continuous and discrete signals.

- Write the equation of sampled signal.

- Explain clc command.

- Narrate the term clear in MATLAB.

- Draw the Frequency response of high  pass filter.

- What is Band stop  pass filter.

- What is band reject filter.

- The instantaneous sampling means?

- The sampling technique that has the minimum noise interference is.

- What Types of analog pulse modulation systems.

- Write about correlation.

- Write about Auto correlation.

- Explain Draw backs in Analog signal processing

- Write the properties of Expectation.

- How to find Mean value using MATLAB.

- What is the Relation between Mean and Mean square value.

- Relate PDF and CDF.

- Write the properties of CDF.

- Write the properties of PDF.

- measure the mean value of Gaussian Density function.

- Differentiate Binomial and Gaussian Distribution.

- Calculate mean value of any distribution function at origin.

- Find Inverse Fourier Transform of impulse function.

EXERCISE PROGRAMS

1.Write a program to simulate parseval’s relation

2. Write a mat lab program to perform the cross correlation between sinusoidal and ramp function and see how mat lab reacts to it.

3. Write a MATLAB program to perform cross correlation between square and step signal and see how mat lab reacts to it.

4. Write a MATLAB program to perform cross correlation between sinusoidal and ramp signal and see how mat lab reacts to it.

5. Write a MATLAB program to perform the cross correlation between X (n) = [1 2 3 5] and y (n) = [- 1 – 2] and see how matlab reacts to it.

6. Write a MATLAB program to perform the cross correlation between X (n) = [1 -3  5] and y (n) = [1 2 3 4] and see how matlab reacts to it.

7. Write a MATLAB program to perform the cross correlation between X (n) = [1 0 1 1] and y (n) = [1 0 0 0 0] and see how matlab reacts to it.

8.Write a MATLAB program to perform the cross correlation between X (n) = [1 1 1 1 0 0 0 0] and y (n) = [1 0 1 0 1 0 1 0] and see how matlab reacts to it.

9.Write a MATLAB program to perform the cross correlation between X (n) = [1 1 0 0] and y (n) = [1 1 1 1] and see how matlab reacts to it.

10.Write a MATLAB program to perform the cross correlation between X (n) = [6 7 8 9 10] and y (n) = [5 4 3 2 1] and see how matlab reacts to it.

11.Write the MATLAB program to perform cross correlation between the following sequences

X(n)=[2 -2 3 2, h(n) = [-2 4 -5 2].

12. Write a mat lab program to perform the convolution between step and ramp function and see how mat lab reacts to it.

13. Write a MATLAB program to perform convolution between square and step signal and see how mat lab reacts to it.

14. Write a MATLAB program to perform convolution between sinusoidal and square signal and see how mat lab reacts to it.

15. Write a MATLAB program to perform the convolution between X (n) = [2 3 4 5] and y (n) = [-2  –4] and see how matlab reacts to it.

16. Write a MATLAB program to perform the convolution between X (n) = [3 -4 6] and y (n) = [2 6 8 6] and see how matlab reacts to it.

17. Write a MATLAB program to perform the convolution between X (n) = [3 1 2 0] and y (n) = [2 4 3 4 5] and see how matlab reacts to it.

18.Write a MATLAB program to perform the convolution between X (n) = [2 2 1 0 2 1 1 0] and y (n) = [1 1 1 2 1 0 1 0] and see how matlab reacts to it.

19.Write a MATLAB program to perform the convolution between X (n) = [2 1 2 1] and y (n) = [1 2 1 2] and see how matlab reacts to it.

20.Write a MATLAB program to perform the convolution between X (n) = [5 6 9 8 7] and y (n) = [1 2 3 2 1] and see how matlab reacts to it.

21.Write the MATLAB program to perform cross correlation between the following sequences

X(n)=[4 -6 5 1], h(n) = [-1 3 -4 3].

22. Write a mat lab program to perform the cross correlation between ramp and step function and see how mat lab reacts to it.

23. Write a MATLAB program to perform cross correlation between square and step signal and see how mat lab reacts to it.

24. Write a MATLAB program to perform cross correlation between sinusoidal and step signal and see how mat lab reacts to it.

25. Write a MATLAB program to perform the convolution between X (n) = [1 5 6 5] and y (n) = [-3 –6] and see how matlab reacts to it.

Real Time Applications:

- Parseval’s theorem for energy signals

- Parseval’s theorem for power signals

- Digital signal processing applications etc…